3GPP TSG-SA3 Meeting #110AdHoc-e 	draft_S3-231924-r3

- e-meeting, 17 - 21 April 2023

- Source:	Huawei, HiSilicon, ZTE (?), ChinaTelecom, Ericsson (?)

- Title:	Update to the KI#2 conclusion in TR 33.740

- Document for:	Approval

Agenda Item:	5.3

# 1	Decision/action requested

Approve the conclusion proposal to TR 33.740

# 2	References

N/A

# 3	Rationale

The contribution proposes to add conclusion to KI#2.

Merger of S3-231844, S3-231868 and S3-231956.

# 4	Detailed proposal

**** Start of Changes****

## 7.2	Key Issue #2: Security of UE-to-UE Relay

For Key Issue #2, the following statements are agreed:

NOTE 1:	The choice and co-existence of the security mechanisms in different use cases (i.e., U2U Relay in and out of coverage) will be decided in normative phase.

Regarding End-to-End security:

- For L2 relay, the source UE and the target UE can establish a PC5 link over the L2 relay after the per hop link is established.

- For L2 relay, the End UEs (Source UE and/or Target UE) reuse the unicast mode security mechanism defined in clause 5.3.3.1 of TS 33.536 [6] to establish a secure connection via the Relay UE.

- For L3 relay, End-to-End security can be supported in the application layer which is out of 3GPP scope.

Regarding hop-by-hop security during PC5 link establishment in Layer 3/Layer 2 UE-to-UE Relay Communication:

- When the Layer 3/Layer 2 UE-to-UE Relay is in 3GPP coverage, the End UE can establish a secure PC5 link with the L3 relay with network assistance. The similar security procedure as PC5 security for 5G ProSe Communication via 5G ProSe Layer-3 UE to-Network Relay as defined in clause 6.3 in TS 33.503 [6] can be reused.

- The solutions for UE-to-UE Relay authorisation and security can be classified as user-plane (UP) or controlled-plane (CP) based solutions. It is concluded that both control plane and user plane solutions are supported for 5G ProSe Layer 2 UE-to-UE Relay and 5G ProSe Layer 3 UE-to-UE relay.

- When the Layer 3/Layer 2 UE-to-UE Relay is out of 3GPP coverage, the End UE can establish a secure PC5 link with the L3 relay without network assistance. The similar security procedure as PC5 security for unicast mode 5G ProSe Direct Communication as defined in clause 6.2.3 in TS 33.503 [6] can be reused.

**** End of Changes****