3GPP TSG-SA3 Meeting #110Ad-Hoc-e 	draft_S3-232042-r3

- Electronic meeting, Online, 17 - 21 April 2023								(revision of S3-232042)

- Source:	Xiaomi

- Title:	33.893: Conclusion on Key Issue #4

- Document for:	Approval

Agenda Item:	5.19

# 1	Decision/action requested

SA3 is kindly asked to approve the proposed conclusion to be added for key issue #4 in TR 33.893.

# 2	References

[1]	TR 33.893 v0.6.0		Study on Security Aspects of Ranging Based Services and Sidelink Positioning

# 3	Rationale

This pCR proposes to conclude on key issue #4 of TR 33.893 [1].

# 4	Detailed proposal

*************** Start of the Change ****************

# 7	Conclusions

7.x	Conclusions on Key Issue #4

For protection of unicast direct communication, the following conclusions are made:

-	For security of SR5 direct communication for Ranging/SL Positioning services provided by application providers, the security mechanisms defined for V2X unicast mode communication in TS 33.536 [5] and for 5G ProSe unicast mode Direct Communication in TS 33.503 [6] are reused.

-	For security of SR5 direct communication for network assisted SL positioning services provided by operators, the security mechanisms defined for 5G ProSe UE-to-Network Relay communication in TS 33.503 [6] can be reused.

-	To protect RSPP unicast messages on SR5 reference point, security policies for the Ranging/SL Positioning service are to be enforced on PDCP layer. control

Editor’s Note: Further conclusion is FFS.

*************** End of the Change ****************