3GPP TSG-SA3 Meeting #110Ad-Hoc-e 	draft_S3-232043-r2

- Electronic meeting, Online, 17 - 21 April 2023								(revision of S3-232043)

- Source:	Xiaomi

- Title:	33.700-28: Update to Key Issue #1

- Document for:	Approval

Agenda Item:	5.25

# 1	Decision/action requested

SA3 is kindly asked to approve the update to key issue #1 in TR 33.700-28.

# 2	References

[1]	S2-2303429				Transfer of Satellite Coverage Data to a UE and AMF

[2]	TR 33.700-28 v0.3.0		Study on security aspects of satellite access

[3]	TR 23.700-28	 v18.0.0	Study on Integration of satellite components in the 5G architecture; Phase 2

# 3	Rationale

According to the current SA2 R18 specification for satellite access with discontinuous satellite coverage agreed in S2-2303429 [1], it is specified that satellite coverage availability information may be provisioned to the AMF by O&M or by AF. The satellite coverage availability information is not UE specific and can be applied by the AMF for any UE in the affected area. The provisioned satellite coverage information is used by the AMF to determine the UE out-of-coverage period.

Due to the updates in SA2, this pCR proposes to update the current key issue #1 in TR 33.700-28 [2] which is based on the obsoleted conclusions in TR 23.700-28 [3].

# 4	Detailed proposal

*************** Start of the 1st Change ****************

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 23.700-28: "Study on Integration of satellite components in the 5G architecture; Phase 2".

[3]	3GPP TS 33.501: "Security architecture and procedures for 5G System".

[x]	3GPP TS 23.501: "System architecture for the 5G System (5GS); Stage 2".

*************** Start of the 2nd Change ****************

## 5.1	Key issue #1: Protection of satellite coverage information received by 5GC/EPC

### 5.1.1	Key issue details

To support mobility enhancement and power saving optimization, multiple solutions were documented in TR 23.700-28 [2], which require 5GC/EPC to move the UE into CM-IDLE state and provide power saving parameters or mobility management parameters properly, when the UE is about to leave satellite coverage. This requires that satellite coverage availability information in the affected area to be made aware of by 5GC/EPC. According to the conclusions in TR 23.700-28 [2] and normative work in 23.501 [x], such satellite coverage availability information could be received by 5GC/EPC from different potential sources, e.g. the OAM, the AF/external server (e.g. Coverage Map Server). While the sources like the OAM could be trusted, the sources like the AF/external server may not always be trusted.

5.1.2	Threats

If 5GC/EPC receives falsified or tampered satellite coverage availability information, the 5GC/EPC may be misled to put a CM-CONNECTED UE into CM-IDLE state when the UE is still in satellite coverage, leading to service interruption; or the 5GC/EPC may be misled to provide inappropriate mobility management parameters and/or power saving parameters to the UE, which fails the optimization of power consumption. Both cases are a type of DoS attack on the UE.

If an AF is not authorized to provide satellite coverage availability information in the affected area, the 5GC/EPC may also suffer from the above threat.

### 5.1.3	Potential security requirements

The 5GS/EPS shall provide a means to ensure that the AF/external server is authorized to provide satellite coverage availability information to 5GC/EPC.

*************** End of the Changes ****************