3GPP TSG-SA3 Meeting #112 	S3-233506

# Goteborg, Sweden, 14 - 18 August 2023

# Source:	3GPP CT1

Title:	LS on Handling of SOR counter and the UE parameter update counter if stored in NVM

# Document for:	Information, Discussion

Agenda Item: 3

3GPP TSG-CT WG1 Meeting #141e	C1-232696

# Online 17– 21 April 2023

# Title:	LS on Handling of SOR counter and the UE parameter update counter if stored in NVM

# Response to:	--

# Release:	Rel-18

# Work Item:	5GProtoc18

Source:	CT1

To:	SA3

Cc:	-

Contact Person:

Roland Gruber

E-mail Address:	rgruber at apple dot com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	none

1. Overall Description:

If Service n°123 is "available" in EFUST on the USIM, then the KAUSF is stored in EF5GAUTHKEYS on the USIM otherwise the KAUSF is stored in non-volatile memory (NVM) of the ME.

If Service n°123 and Service n°133 are "available" in EFUST on the USIM, then the SOR counter and the UE parameter update counter are stored in EF5GAUTHKEYS on the USIM otherwise the SOR counter and the UE parameter update counter are stored in NVM of the ME.

If the KAUSF is stored on the USIM and the SOR counter and the UE parameter update counter are not present on the USIM, then the SOR counter and the UE parameter update counter stored in NVM are considered as valid after power up, if the SUPI stored in NVM of the ME is equal to the SUPI of the inserted USIM.

However, the USIM could have been used in a different ME-2 in the meantime and a new authentication could have been triggered in the ME-2. In this case if the USIM is re-inserted in ME-1, ME-1 would apply the outdated previously stored counters, i.e., the counters stored in the NVM of ME-1 related to the previous KAUSF might be higher than the counters used in the context of the new KAUSF. In consequence new SOR messages would be ignored due to outdated counter values store in NVM of ME-1.

2. Actions:

To SA3 group.

ACTION: 	CT1 kindly asks SA3 to study the problem outlined above and take the appropriate actions.

3. Date of Next CT1 Meetings:

CT1#142	22nd – 26th May 2023 		Bratislava

CT1#143	21st  – 25th August 2023 		Goteborg

CT1#144	9th   – 13th October 2023 		China (TBC)

CT1#145	13th   – 17th November 2023 		Chicago