3GPP TSG-SA3 Meeting #115	S3-240783

Athens, Greece, 26th February - 1st March 2024

Title:	[Draft] Reply LS on clarification on home network triggered re-authentication

Response to:	LS S3-240220/C4-235577 on LS on clarification on home network triggered re-authentication

Release:	Rel 18

Work Item:	HN_Auth

Source:	Xiaomi [to be SA3]

To:	CT4

Cc:	-

Contact person:	Jouy Shang

shangzhengyi@xiaomi.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

SA3 would like to thank CT4 for their LS on Clarification on home network triggered re-authentication. SA3 would like to provide the following feedback.

1. When the (source) AMF receives a Reauth Notifications request, there is an ongoing HO/Mobility registration without AMF change.

2. When the (source) AMF receives a Reauth Notifications request, there is an ongoing HO/Mobility registration with AMF change.

3. When the target AMF receives a Reauth Notifications request, there is an ongoing HO/Mobility registration with AMF change.

4. When the AMF receives a Reauth Notifications request, there is an ongoing authentication.

5. When the AMF receives a Reauth Notifications request, the UE is unreachable and the AMF doesn’t know whether there is another AMF available over the other access type.

Q1. Should the above user cases be considered as valid failure cases?

A1. The above cases can be considered as valid failure cases. In addition, SA3 has identified another failure case: the HONTRA procedure is not allowed by the local authentication policy.

Q2. Does SA3 see any need of differentiated handling in the UDM in each failure case? Or, what’s the expected UDM behaviour from the SA3 point of view?

A2. The action of UDM should be differentiated based on the returned failure case.

For failure cases 1-3, the UDM can determine later on whether to trigger the primary authentication by resending Nudm_UECM_Re-AuthenticationNotification message.

For failure case 4, the UDM can acknowledge the ongoing of HONTRA procedure.

For failure case 5, the UDM can check if another AMF is available over the other access and resend the Nudm_UECM_Re-AuthenticationNotification message to the AMF over the other access.

For the new failure case: if the HONTRA procedure is not allowed by the local authentication policy, the UDM can acknowledge the failure of HONTRA procedure.

2. Actions:

To CT4

ACTION: 	SA3 kindly asks CT4 to take the above information into account.

3. Date of Next SA3 Meeting:

SA3#115AdHoc-e	15 - 19 April 2024	Electronic meeting

SA3#116	20 - 24 May 2024		Jeju (South Korea)

SA3#117	19 - 23 August 2024		Maastricht (Netherlands)