3GPP TSG-SA3 Meeting #116	S3‑242586

Jeju, South Korea,  20th - 24th May 2024								S3-241811

# Source:	Nokia

# Title:	pCR on solution for VFL member authorization.

# Document for:	Approval

Agenda Item:	5.13

# 1	Decision/action requested

It is requested to approve the KI

# 2	References

[i] 3GPP TR 33.784: "Study on security aspects of Core Network Enhanced Support for AIML"

[ii] 3GPP 23.700-84: “Study on Core Network Enhanced Support for Artificial Intelligence (AI)/Machine Learning (ML)”

# 3	Rationale

This pCR proposes a solution for KI in TR 33.784

# 4	Detailed proposal

**** START OF CHANGE ****

## 6.Y	Solution #Y: Solution for VFL member authorization

### 6.Y.1	Introduction

This solution addresses KI#2.

The concept of the solution is similar to ‘X.9 Authorization of selection of participant NWDAF instances in the Federated Learning group’ defined in clause X.9 of TS 33.501 [5].

In the VFL case, NWDAF and/or AF can participate in the VFL process. Please note that internal AF information is available at the NRF as a part of AF's NFprofile and external AF information is also available at the NRF as a part of NEF NFprofile. Please refer to the section 6.1.6.2.48 NefInfo of TS 29.510 [x] that contains pdfData that contains AF connected via NEF.

### 6.Y.2	Solution details

- NWDAF/Internal AF or NEF (on behalf of external AF) updates their NFprofile in the NRF with VFL information, including its VFL capabilities (e.g. privacy protection supported/not supported, supported role (Client/Server/Active/Passive, etc.)), interoperability indicator per analytics. The NFprofile also includes allowed VFL capability per consumer.

- The consumer NWDAF/AF or NEF (on behalf of external AF) sends an access token request to the NRF. The access token request may contain the Analytics ID, source NF, interoperability indicator/vendor Id and target NF's VLF capability for the requested VFL process.

- The NRF authorises the NWDAF/Internal AF or NEF & external AF based upon the information received in Step 1. If the authorization succeeds, NRF generates the access token with the claim, including the target VFL capability.

- The consumer NWDAF/Internal AF or NEF (on behalf of external AF) sends the service request, including an access token, to the producer NWDAF/Internal AF or NEF. The producer verifies the token and allows the VFL procedure.

Editor’s Note: Capability and privacy protection details are FFS. Why VFL capability is per consumer is FFS.

Editor’s Note: Whether the NRF authorizes the NEF or external AF behind the NEF is ffs. Secondly when NEF registers AF details are FFS.

Editor’s Note: Rational for adding target VFL capability into token and how to verify it is ffs.

### 6.Y.3	Evaluation

Editor’s Note: Each solution should motivate how the potential security requirements of the key issues being addressed are fulfilled.

TBD

**** END OF CHANGE ****