3GPP TSG-SA3 Meeting #117	S3-243087

Maastricht, Netherlands  19 - 23 August 2024

# Source:	Huawei, HiSilicon

# Title:	Evaluation for usecase #4 on KI#1

# Document for:	Approval

Agenda Item:	5.1

# 1	Decision/action requested

Inclusion of Evaluation for usecase #4 on KI#1 in TR 33.794 [1].

# 2	References

[1]	3GPP TR 33.794: "Study on enablers for Zero Trust Security".

# 3	Rationale

To consider the inclusion of evaluation for usecase 4 on KI#1 to TR 33.794 [1].

# 4	Detailed proposal

*****Start of Changes*****

### 5.1.4	Use case #4:  Reconnaissance

#### 5.1.4.1	Description

Secure communications between NFs and with other NFs and the NEF nodes is essential. TLS is specified to secure the transport layer (See 3GPP TS 33.501 [4] sub-clause 9.5, 12.3, 13.1.0). When a TLS connection is setup both sides of the TLS connection check to ensure that the certificate is valid and has not been revoked; however, no validation is performed to ensure that the NF setting up the TLS connection is

- expected to communicate with the NF terminating the TLS connection (e.g., No validation is performed on other parameters e.g. subjectAltName defined in 3GPP 33.310 [6]); or

- Performing API call(s).

A compromised NF can setup TLS connections to any number of other entities, collect the TLS certificates of the other NFs and use  the data gathered at a later date to assist in performing other attacks.

Not monitoring or collecting data related to successful NF TLS connections can reduce the ability to detect key indicators of potential compromise of NFs.

Analysis of security events lacks trustworthy information regarding the potential source of adversity.

#### 5.1.4.2	Relevant data

Source IP address;

TLS certificate of the NF consumer;

APIs invoked via the TLS connection; and

If no APIs where invoked, the length of time the TLS connection was established for, or what point in the TLS establishment procedure it was terminated.

#### 5.1.4.3	Evaluation of the identified data

TLS connections that are not fully established, or TLS connections that are established and no APIs are used should be notified to the Operators Security Function. Both of these are abnormal behaviour as if a TLS session is setup, one would expect at least one API call. Example information that could be useful includes the source IP address, TLS certificate of the NF consumer, timestamp when the event occurred, and the duration of the event, what and if any API calls were made.

NOTE:	Some of the data identified above might not be available to the SBA layer.

The security requirements outlined in TS 33.501[4] mandate mutual TLS across the 5G Service-Based Architecture (SBA) to ensure only authorized clients can establish connections, So Network Functions (NFs) receive digital certificates following identity verification by the operator's Certificate Authority (CA) during the Certificate Signing Request (CSR) procedure. During TLS negotiation, NFs validate these certificates by checking the certificate chain, expiration dates, and revocation status, ensuring proper identification before TLS connection establishment. It is a misconception that no validation occurs before setting up TLS, extensive verification steps are involved.

Collecting peer certificates during TLS negotiation phase alone does not facilitate attacks or indicate malicious intent also mutually authenticated TLS connections do not inherently imply malicious intent if terminated without API use, as such scenarios can relate to network redundancy and high availability designs.

The concept of a "partially established" TLS connection is not recognized, as TLS connections are either established or not.

*****End of Changes*****