3GPP TSG-SA3 Meeting #120	S3-250374

Athens, Greece 17 – 21 Feburary 2025

Title:	[draft] Reply LS on security questions related to NAS layer overhead reduction for data transfer using control plane CIoT

Response to:	C1-247156 LS on security questions related to NAS layer overhead reduction for data transfer using control plane CIoT

Release:	Rel-19

Work Item:	NORDAT-CP

Source:	vivo (to be SA3)

To:	CT1

Cc:

Contact person:	Li Hu

huli@vivo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1	Overall description

SA3 thanks CT1 for the LS C1-247156 on security questions related to NAS layer overhead reduction for data transfer using control plane CIoT.

After discussion, SA3 has the answers for the following questions raised by CT1:

Question 1: Currently, SERVICE REQUEST message in EPS NAS uses short MAC and short sequence number. Would it be acceptable to use short MAC and/or short sequence number for the new NAS message format with minimized overhead for data transfer via NAS?

Answer 1: The use of a short sequence number is acceptable for synchronization purposes, as the MAC input will utilize the complete 32-bit NAS COUNT. However, employing a short MAC may compromise integrity strength and is therefore not recommended if data is transferred.

Question 2: Currently, all initial NAS messages contain Key Sequence Indicator (KSI). Would it be acceptable not to include the KSI in the new initial NAS message format with minimized overhead for data transfer via NAS?

Answer 2: KSI is used to indicate the native KASME. Given that both the UE and MME may hold multiple KASME, it is necessary to include the KSI in the NAS message.

Question 3: In the NAS message format with minimized overhead for data transfer via NAS, only a portion of the message will be ciphered. Is it acceptable for the ciphered portion not to be byte-aligned? Not having to byte-align the ciphered portion would allow for more efficient message encoding.

Answer 3: The EEA does not require messages to be byte-aligned since the LENGTH can be set to bit length as illustrated in section Annex C1 of TS 33.401.

2	Actions

To CT WG1:

ACTION: 	SA3 kindly asks 3GPP CT1 to take the above information into consideration

3	Dates of next TSG SA WG 3 meetings

SA3#121				        7-11 April 2025				   Goteborg, Sweden

SA3#122				        19-23 May 2025				       Fukuoka, Japan