3GPP TSG-SA3 Meeting #120	S3-250438

Athens, Greece, 17 - 21 February 2025

# Source:	Qualcomm Incorporated

# Title:	U2U relay communication security

# Document for:	Approval

Agenda Item:	4.15

# 1	Decision/action requested

This contribution propose text for U2U relay communication security.

# 2	References

[1]	TR 33.743

[2]	Skeleton for the draft CR

# 3	Rationale

This contribution proposes text for U2U relay communication security based on the conclusions in the TR 33.743 [1].

# 4	Detailed proposal

It is proposed that the below changes to the draft CR [2] are approved.

# * * * * First change * * * *

### 6.6.XX	Security for 5G ProSe Communication via 5G ProSe Layer-3 Multi-hop UE-to-UE Relay

This clause describes the security requirements and the procedures for 5G ProSe Communication via 5G ProSe Layer-3 Multi-hop UE-to-UE Relay defined in clause 6.7.5 of TS 23.304 [2].

#### 6.6.XX.1	Security requirements

The 5G System shall provide a means for confidentiality protection, integrity protection and replay protection of communication messages in the multi-hop UE-to-UE Relay communication.

The 5G System shall provide a means for authentication and authorization of the UEs in multi-hop UE-to-UE Relay communication.

The 5G System shall provide a means for mitigating trackability and linkability attacks on UEs in multi-hop UE-to-UE Relay communication.

#### 6.6.XX.2	Security procedure for 5G ProSe Layer-3 Multi-hop UE-to-UE Relay communication

Editor’s Note:	This clause describes the Security procedure for 5G ProSe Layer-3 Multi-hop UE-to-UE Relay communication of IP PDU type, and for 5G ProSe Layer-3 Multi-hop UE-to-UE Relay communication of non-IP PDU type.

###### 6.6.XX.2.XXX	Security of 5G ProSe Layer-3 Multi-hop UE-to-UE Relay communication for IP type

The 5G ProSe Layer-3 Multi-hop UE-to-UE Relay communication consists of two types of PC5 link establishment: one for PC5 link establishment among 5G ProSe UE-to-UE Relays and the other one for PC5 link establishment between an 5G ProSe End UE and 5G ProSe UE-to-UE Relay as specified in clause 6.7.5.2.1 of TS 23.304 [2].

For both types of PC5 link establishment, the security procedure for unicast mode 5G ProSe Direct Communication specified in clause 6.2 is reused with the following modifications:

-	The RSC is included in the DCR message.

-	The DCR message is protected based on the security mechanism defined in clause 6.3.5 with a modification that the UP-PRUK ID/CP-PRUK ID is not used in clause 6.3.5.2.

# * * * * End of change * * * *