3GPP TSG-SA3 Meeting #120	S3-250478

Athens, GREECE, 17-21 February 2025

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.122 | CR | 0085 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CAPIF-8 - Authorization for finer level service API access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | CAPIF_Ph3_sec |  |  |  |  |  | Date: |  |  | 2025-02-21 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Security details for CAPIF-8 were out of scope in Rel-18 and need to be addressed in Rel-19. |  |  |  |  |  |  |  |  |
| Summary of change: |  | CAPIF-8 solution on finer level of granularity. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No security solution for CAPIF-8 on finer level of granularity. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.Y (new), 6.Y.1 (new), 6.Y.2 (new), 6.Y.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

******* START OF CHANGES

## 6.Y	Authorization for finer level service API access

For finer level service API access, the authorization request includes the indication of the requested service operation/resource. For RNAA, the request also includes the GPSI of the UE.

CCF verifies the API invoker ID and the requested service operation/resource, if available. The authorization result is sent to the API invoker if the verification is successful. The API Invoker includes the required additional information to CCF during the Access token Request.

NOTE 1: A finer granularity can include: Resource Owner ID, operations (e.g. retrieve, create, etc), features (e.g. feature 1, feature 2, etc) and resources (e.g. resource 1, resource 2, etc).

The API Invoker will include the additional authorization details in the scope parameter to allow to distinguish both the resource owner and the resources that the API Invoker intends to access. This will allow the CCF to verify if the resource owner allowed this specific API Invoker to access the requested resources.

When the verification is completed, the CCF will include the authorization details, with the new granularity, into the access token returned to the API Invoker.

The previously provided access token will allow the AEF to correctly authorize, or deny, the request by the mechanism already available to AEF. AEF must be able to verify the details for finer granularity access control.

NOTE 2: If finer level granularity is used, the inclusion of resource owner identifier is a scope parameter while in Rel-18 RO ID is part of claim, which would be left empty in this case. This allows to distinguish between Rel-19 and Rel-18 to avoid backward compatibility problems.

******* END OF CHANGES