3GPP TSG-SA3 Meeting #120	S3-250510

# Athens, Greece 17 – 21 February 2025

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.122 | CR | draft | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resource owner authorization management for CAPIF_Ph3_Sec |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | CAPIF_Ph3_Sec |  |  |  |  |  | Date: |  |  | 2025-2-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This draft CR propose to define the Resource owner authorization management in TS 33.122 based on the conclusion of KI #1.2 in TR 33.700-22. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Capture the security procedures for resource owner authorization management. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | New CAPIF phase 3 features resource owner authorization will be unworkable. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.3.4, 6.5.3.B (new clause) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Change

6.5.3	Authentication and authorization for RNAA

6.5.3.1	General

The authorization function shall obtain the necessary permission from the resource owner for allowing the API invoker to access a northbound API.

RNAA shall use token-based authorization using OAuth 2.0 framework with the following roles:

-	The API invoker has the role of the OAuth 2.0 client.

-	The CCF has the role of the OAuth 2.0 authorization server, i.e., providing the access token used for RNAA.

-	The AEF has the role of the resource server.

The access tokens used for RNAA shall contain the resource owner ID.

The resource owner may be the user of the UE or the owner of the subscription depending on the use case and regulations.The resource owner ID is specified as the GPSI of the corresponding UE if the resource is related to a UE.

NOTE: The present document does not specify the resource owner.

The access token shall include the resource owner ID and the API invoker ID. The resource owner ID is the GPSI. The API invoker ID binds the token to the API invoker. To avoid privacy issues, GPSI should be different from MSISDN, SUPI etc.

The AEF shall check if the token includes resOwnerId claim, which includes resource owner ID, to identify that it is a token used in RNAA.

AEF shall do the authorization check of the API invocation request for accessing the resources of the resource owner. AEF checks the request against the token, including:

1) checking the token integrity and

2) checking whether the GPSI (if present) in the API invocation request is compliant with the resource owner ID in the access token. As the token includes resource owner ID, there is no need for additional UE authentication in API invocation. Moreover, the token should be able to restrict the API invoker to a specific resource (e.g., location, QoS, PDN connectivity status) of the resource owner.

For OAuth 2.0 flows involving redirection, authentication between CCF/AUF and UE should be performed after API Invoker redirects the UE to CCF/AUF.

In case of an external AF (i.e., not the application on the UE) being the API invoker, for mutual authentication of API invoker AF and API exposing function, the authentication methods of clause 6.4 and clause 6.5.2 are reused.

For authorization, the following OAuth 2.0 flows may be used:

-	Client credential flow (according to RFC 6749 [4]),

-	Authorization code flow (according to RFC 6749 [4]), or

-	Authorization code flow with PKCE (according to RFC 7636 [11]).

CCF shall indicate the selected flows to the API invoker.

CCF shall give service authorization which subscribers or users can use RNAA.

For selecting the authorization method, the procedure as specified in clause 6.3.1.2 is used with the following RNAA specific additions. The API invoker shall include in the Security Method Request the supported RNAA authorization flows. The CCF shall determine the RNAA authorization flow based on the RNAA capabilities of the CCF, AEF, and API invoker. The API invoker shall use the determined RNAA authorization flow in the subsequent communication with the CCF and AEF.

NOTE: In the present document, only a UE accessing its own resources is considered if the API invoker is on a UE.

6.5.3.2	Authorization using oauth client credential flow

If client credential flow is used for authorization of the API invoker by the AEF, the procedures in RFC 6749 [4] shall be followed with the following profile:

-	The access token request message may include the resource owner ID.

NOTE 1: If the API invoker is on a UE, the CCF obtains its GPSI during authentication.

Editor’s note: the mapping of API Invoker ID and GPSI is left for stage 3.

-	The CCF shall check whether the API invoker is entitled to consume the API and allowed to access the resources of the resource owner, by using authorization information available in the CCF.

-	If the API invoker is on a UE, the CCF shall check that the UE is accessing its own resources. If the API invoker is an AF not on a UE, the check is omitted.

NOTE 2: How to get the authorization from the resource owner and store it in the CCF is out of scope of the present document.

6.5.3.3	Authorization using authorization code (optional PKCE) flow

If authorization code flow, optionally with PKCE, is used by the AEF for authorization of the API invoker, the procedures in RFC 6749 [4] and optionally RFC 7636 [11] shall be followed, with the following profile:

-	The authorization token and/or authorization request may include the resource owner ID.

NOTE: If the API invoker is on a UE, the CCF obtains its GPSI during authentication.

Editor's note: the mapping of API Invoker ID and GPSI is left for stage 3.

-	The resource owner dynamically authorizes the API invoker to access the resource owner's resources as described in RFC 6749 [4] and optionally RFC 7636 [11].

-	If the API invoker is on a UE, the CCF shall check that the UE is accessing its own resources. The access token shall contain the resource owner ID (i.e. GPSI) and the API invoker ID. If the API invoker is an AF not on a UE, the check is omitted.

6.5.3.4	Revocation

Figure 6.5.3.4-1 illustrates the procedure for revoking API invoker authorization initiated by resource owner in RNAA scenarios.

Figure 6.5.3.4-1: Procedure for revoking API invoker authorization initiated by resource owner

- Mutual authentication between the authorization server and the resource owner is completed.

- The resource owner sends resource owner authorization revocation request to the CCF via the UE, resource owner function, web page etc. The resource owner authorization revocation information in request message includes the Resource Owner ID, service API information, API invoker ID and addition information.

Editor's note: It is ffs whether addition information is need.

- The CCF determine the details of the API invoker and the service API used to identify the RNAA-related revoked token based on the received resource owner authorization revocation information.

- The CCF sends resource owner authorization revocation request message to the AEF as defined in clause 8.23.4 of TS 23.222 [3] with the resource owner authorization revocation information to identify the RNAA-related revoked token.

- The

- AEF, storing the resource owner authorization revocation information about the RNAA-related revoked token, shall invalidates the API invoker authorization corresponding to the RNAA-related revoked token. The AEF shall check whether the token presented by an API invoker is revoked or not, before responding to the API invoker’s invocation request.

- The AEF sends a revoke API invoker authorization response to the CAPIF core function.

- The CAPIF core function invalidates the API invoker authorization corresponding to the service API.

- The CCF sends a resource owner authorization revocation response to the resource owner via the UE, resource owner function, web page etc.

- The CCF provided notification message to the API invoker shall include the information to identify the RNAA-related revoked token.

6.5.3. B	Resource owner authorization management

Resource owner authorization information/authorization revocation information is transferred between the ROF and the CCF via secure CAPIF-8 reference point.

The resource owner is authenticated before accepting resource owner authorization and authorization revocation information.

NOTE: How to authenticate the resource owner is left to implementation.

Resource owner authorization information includes Resource Owner ID, service API information, API invoker ID and addition information.

Resource owner authorization revocation information includes Resource Owner ID, service API information, API invoker ID and addition information.

Editor's note: It is ffs whether addition information in Resource owner authorization information/authorization revocation information is need.

End of Changes