3GPP TSG-SA3 Meeting #120	S3-250527

# Athens, Greece, 17 -21 February 2025

Source:	OPPO

Title:	Update AIOT sol#7

Document for:	Approval

Agenda item:	5.9

Spec:	3GPP TR 33.713

Version:	0.6.0

Work Item:	FS_Ambient_IoT_Sec

Comments

This contribution proposes updates for AIoT security solution #7.

Proposed change 1: According to the lasted agreement of RAN2, there is meant to avoid duplicate response from the device to a reader.  Thus, the  SEQNO is not needed and the related ENs can be deleted.

Editor's Note:	The impact of removing the use of SEQNO (e.g., replay against device or network) from AKA is FFS

Agreements

- The A-IoT paging message can include information to avoid duplicate response from the device to a reader.  This information should be short and simple.  FFS how to indicate.  Wait for further information and requirements from other WGs and make this decision in normative phase.

- Based on this information the device determines whether to skip sending the response to paging.

- From RAN2 perspective it is feasible to support paging multiple device IDs, however depending on TB size it may not be possible to include multiple device IDs.    RAN2 will determine the need for this after considering aspects related to RAN2 multiplexing and other WGs.

- If multiple device IDs in single paging is supported, if A-IoT paging message contains multiple device IDs, reader can configure either contention free RA or contention-based RA

Proposed change 2: AK in this solution is not used to protect SEQNO. AK in this solution is used to make sure the input parameters are different for MAC and RES. So, the related ENs can be deleted

Editor's Note:	Anonymity key (AK) is used to protect SEQNO in AKA. If SEQNO is not used, the necessity of using AK to compute MAC and XRES is FFS.

Proposed change 3: This solution is for “inventory and command” case, so at least before command is sent, the AIoT NF triggers the authentication. Thus, related text was added to step 1, and the following EN is deleted.

Editor's Note:	When the AIoT NF triggers the authentication is FFS.

Proposed change 4: It is added “Function 1 and Function 2 can be for example f1 and f2”, so the related ENs can be deleted.

Editor's Note:	Further evaluation of the usage of the crypto primitives is FFS.

Editor's Note:	The security level of MAC/RES calculation compare to 5G-AKA is FFS.

Proposed change 5: It is added “Note: The storage of key is platform implementation issue” and “Note: The storage of authentication parameters is platform implementation issue” in the solution, so the related ENs can be deleted.

Editor's Note:	It is FFS if the shared key needs confidentiality/integrity protection against physical attack in the device

Editor's Note:	The intermediate authentication parameter storage in AIoT device is FFS.

Proposed change 6: according to SA2 23-700-13 v130, “The AIOTF checks whether the AIoT Device ID from AIoT Device has a subscription and retrieves.” The impact for this solution is AIoT NF can be co-located with AIoT authentication Function, and step 5 can be skipped.

Proposed change 7: according to SA2 23-700-13 v130, “When the AIoT device profile data is managed by the 5GC, it is stored in the ADM (Ambient IoT Data Management), possibly together with UDR, which exclusively supports AIoT devices.” The impact to this solution is to replace UDM with ADM.

Proposed Changes

* * * First Change * * * *

6.7			Solution #7: Lightweight AIoT Authentication solution

6.7.1			Introduction

The assumption of this solution is AIoT device and network sharing a long-term root key K[x].

Note: The storage of key is platform implementation issue.

MAC/XMAC is used for device authenticating network. MAC is calculated by network side with the K, XMAC is calculated by AIoT device side with the K.

Optionally, RES/XRES is used for network authenticating device. XRES is calculated by network side with the K, RES is calculated by device side with the K.

To provide the freshness of the authentication vectors, RAND is also used to calculate MAC/XMAC and optionally RES/XRES.

6.7.2			Details

MAC/XMAC is used for device authenticating network. MAC is calculated by network side with the K, XMAC is calculated by AIoT device side with the K.

Optionally, RES/XRES is used for network authenticating device. XRES is calculated by network side with the K, RES is calculated by device side with the K.

To provide the freshness of the authentication vectors, RAND is also used to calculate MAC/XMAC and optionally RES/XRES.

Similar with 5G-AKA, and based on RAN2 agreement for “inventory and command” case[6], AIoT authentication solution has the following steps:

Figure 6.7.2-1 AIoT Authentication based on 5G-AKA

- AIoT NF sends authentication vector request to ADM for AIoT device. At least before command is sent, the AIoT NF triggers the authentication.

- ADM calculates MAC with K (e.g. the root key of AIoT device) and RAND.

- Optionally, ADM calculates XRES with K and RAND, if network wants to authenticate AIoT device.

- ADM sends RAND, MAC, device ID and optionally XRES to AIoT Authentication Function. AIoT Authentication Function can be co-located with AIoT NF.

- AIoT NF sends Authentication Request including RAND, MAC, device ID to Reader.

- Reader sends Authentication Request including RAND and MAC to AIoT device. Authentication Request is carried by Step C in the “inventory and command” case.

- AIoT device calculates XMAC with RAND and K.

- Optionally, AIoT device calculates RES with K and RAND.

- AIoT device verifies XMAC=MAC, then the network authentication is successful. If there is command, only after successful verification, the command can be proceeded.

Note: The storage of intermediate authentication parameters is platform implementation issue.

- AIoT device sends Authentication Reponses to Reader, this message optionally including RES if network wants to authenticate AIoT device. Authentication Reponses is carried by Step D in the “inventory and command” case.

- Reader sends Authentication Reponses to AIoT NF.

- Optionally, AIoT NF sends Uplink Authentication Request including RES to AIoT Authentication Function.

- Optionally, AIoT Authentication Function verifies XRES=RES, then the AIoT device Authentication is successful.

As an example, the generation of MAC/XMAC and RES/XRES is described as below. Function 1 and Function 2 can be for example f1 and f2.

Figure 6.X.2-2 The generation of MAC and XRES at network side.

Figure 6.X.2-3 The generation of MAC and XRES at AIoT device side.

6.7.3			Evaluation

This solution fulfils the requirements of KI #5 for inventory and command case.

For inventory and command (e.g. Read/Write/Disable) cases, AIoT Device is able to verify the network, thus mitigate forge command to the AIoT Device.

For inventory and command (e.g. Read) case, network is able to verify the AIoT Device, thus mitigate forge data from the AIoT Device to the network.

* * * End of Changes * * * *