3GPP TSG-SA3 Meeting #120	S3-250547

# Athens, Greece, 17 - 21 February 2025

Source:	ZTE Corporation

Title:	Pseudo-CR on Backhaul Link Protection

Document for:	Approval

Agenda item:	4.16

Spec:	3GPP TS 33.545

Version:	0.0.0

Work Item:	5G_Femto_Sec

Comments

Update Backhaul Link Protection enhancements of NR Femto according to TR conclusions.

* * * First Change * * * *

5.4	Backhaul Link Protection

The backhaul link security mechanisms specified in TS 33.320 [x] clause 4.4.5 and clause 7.4 can be derived with the following change:

-	Replace H(e)NB with NR Femto.

When NR Femto GW is needed, the protection of the interface between SeGW and NR Femto GW can based on NDS/IP as specified in TS 33.210 [y]. The protection of the interface between NR Femto GW with other 5GC function can reuses the protection for N2 or N3 as defined in TS 33.501[z].

When NR Femto GW is not needed, the protection of the interface between SeGW and the function in 5GC can be the same as N2 or N3 as defined in clause 9 in TS 33.501[z].

* * * End of Changes * * * *