3GPP TSG-SA3 Meeting #120	S3-250622

# Athens, Greece, 17 - 21 February 2025

Source:	China Mobile

Title:	Pseudo-CR on Lightweight authentication and privacy protection for inventory and command procedure

Document for:	Approval

Agenda item:	5.9

Spec:	3GPP TS/TR 33.713

Version:	0.6.0

Work Item:	FS_AIOT_Sec

Comments

It proposes a new solution for authentication and privacy protection, to mitigate the writing risk that RAN1 indicates.

"An Ambient IoT device will be able to update its non-volatile memory at some point after receiving a trigger from the Reader. Writing may not always be possible at all times."

* * * First Change * * * *

## 6.0	Mapping of solutions to key issues

Table 6.1-1: Mapping of solutions to key issues

| Solutions | KI#1 | KI#2 | KI#3 | KI#4 | KI#5 |
| --- | --- | --- | --- | --- | --- |
| 1 | X |  |  |  |  |
| 2 |  | X |  |  |  |
| 3 |  | X |  |  |  |
| 4 |  |  |  |  | X |
| 5 | X |  |  |  |  |
| 6 |  |  |  |  | X |
| 7 |  |  |  |  | X |
| 8 |  |  |  |  | X |
| 9 |  |  |  |  | X |
| 10 |  |  |  |  | X |
| 11 |  |  | X | X | X |
| 12 |  |  | X | X | X |
| 13 |  |  | X | X | X |
| 14 |  |  |  | X |  |
| 15 |  |  |  | X |  |
| 16 | X |  |  |  |  |
| 17 | X |  |  |  |  |
| 18 |  | X |  |  |  |
| 19 |  | X |  |  |  |
| 20 |  |  | X |  |  |
| 21 |  |  | X |  |  |
| 22 |  |  | X |  | X |
| 23 |  |  | X |  |  |
| 24 |  |  | X |  |  |
| 25 |  |  | X |  |  |
| 26 |  |  | X |  |  |
| 27 |  |  | X |  |  |
| 28 |  |  | X |  |  |
| 29 |  |  | X |  |  |
| 30 |  |  | X |  |  |
| 31 |  |  | X |  |  |
| 32 |  |  |  | X | X |
| 33 |  |  |  | X |  |
| 34 |  |  | X |  |  |
| 35 |  |  | X | X | X |
| 36 |  |  |  |  | X |
| 37 |  |  |  |  | X |
| 38 |  |  | X |  | X |
| 39 |  |  |  |  | X |
| 40 |  |  |  | X | X |
| 41 | X |  |  |  | X |
| 42 |  |  |  | X | X |
| 43 |  |  |  |  | X |
| X |  |  | X |  | X |

* * * Next Change * * * *

## 6.X	Solution #X: Lightweight authentication and privacy protection for inventory and command procedure

### 6.X.1	Introduction

This solution addresses key issues on authentication and privacy protection (KI#3 & KI5).

As indicated that AIoT device may not always write successfully at all times after receiving triggers, it implies that AIoT device may be failed to get parameters from message after triggering. That may cause authentication failure.

This solution is proposing a solution with no parameter after triggering. It based on pseudo-identifiers to make network authentication on AIoT device side, and provide parameter to network for further authentications. As a result, it can provide one-time pseudo-identifiers for each triggering. It means privacy protection for AIoT device also.

### 6.X.2	Solution details

Figure 6.X.2-1: Protection for inventory and/or command procedure

Pre-condition:

The AIoT device and AIoT authentication Function shares same identity of AIoT Device as ID, pre-shared root key as K, and a pre-shared random number as nonce.

AIoT device and AIoT authentication Function calculate pseudo-identity separately as

pID=E(ID, K, nonce),

which  E means to encryption operation.

Note: To distinguish, in AIoT device side it can be marked as pId=E(ID, K, nonce), which should be the same result as pID in authentication Function side.

Procedure:

- When AIoT NF wants to trigger inventory or command procedure, it sends authentication request to Authentication Function procedure with specific AIoT device ID, to retrieve its pseudo-identity.

- AIoT authentication Function responds with pseudo-identity pID in authentication response.

- The AIoT NF tries to trigger AIoT device through paging-like procedure by using pID.

- As only the specific AIoT device owns same pId, only this device can be correctly triggered.

- After that, the device generates random numbers used for device authentication as Rand.

5a. Optionally, if AIoT device needs to update its pseudo-identity based on network indicationg or local policy, it can generate another random number for next round as nonce'. Then device generates next round pseudo-identity pId1=E(ID, K, nonce').

NOTE: How frequent to update pseudo-identity is a tradeoff between security and device cost.

- The AIoT device responds with Rand, encrypted E(Rand), and optional encrypted E(nonce') if available.

- When AIoT NF receives response, it verifies whether Rand can be equal to D(E(Rand)), which D means decryption operation.

- After that, if E(nonce') is available, AIoT NF forwards E(nonce') to AIoT NF authentication Function.

- When authentication Function gets nonce' from E(nonce'), it calculates pseudo-identity pID1=E(ID, K, nonce') for next round authentication.

### 6.4.3	Evaluation

This solution assumes there are pre-configured K as the authentication credential and random nonce stored in AIoT device and network side securely.

As  only specific device and AIoT authentication Function own same K and nonce, no other entity can generates pID/pId. Thus mutual authentication can be achieved through this pseudo-identity. At same time, privacy protection can also be fulfilled as pseudo-identity can be changed. Thus replay attack can also be mitigated.

It solves the writing problem raised by RAN1 because there is no parameter transferred from network to device.

* * * End of Changes * * * *