3GPP TSG-SA3 Meeting #120	S3-250628

# Athens, Greece, 17 - 21 February 2025

Source:	China Mobile

Title:	Pseudo-CR on Location security support

Document for:	Approval

Agenda item:	4.16

Spec:	3GPP TS 33.545

Version:	0.0.0

Work Item:	5G_Femto_Sec

Comments

This contribution proposes the support for location security.

Proposed Changes

* * * First Change * * * *

5.3	Location Security

The location verification security mechanisms specified in TS 33.320 [x] clause 8.1 can be derived with the following changes:

-	Replace H(e)NB with 5G NR Femto.

-	Replace H(e)MS with 5G NR Femto MS.

The 5G NR Femto MS acts as the verifying node.

Optionally the SeGW acts as the verifying node to perform the location verification for the NR Femto node. The location verification is performed during or after the device authentication process. If the location verification fails,  the connection from the NR Femto node to 5GC should be blocked as early as possible.

When the SeGW acts as the verifying node, the mutual authentication between 5G NR femto and SeGW is followed by location verification for the Femto node. If the mutual authentication is IKEv2 certificate-based, an example call flow of the location verification during the authentication process is performed as shown in Figure x. As IKEv2 allows the inclusion of information data into Notify Payload, the location information of the Femto node may be carried in the Notify Payload (see Figure x) during IKEv2 procedures from the 5G NR Femto to the SeGW.

Figure x: Location verification during the mutual authentication process by SeGW

0a.The location information and verification policy is pre-configured in SeGW.

0b.Optionally the location information and verification policy is dynamically provided by NR Femto management system or NF in 5GC.

1-3.The same as the steps 1-3 in TS 33.320[x] clause A.1.

4.	The same as the step 4 in TS 33.320[x] clause A.1. Additionally, the NR Femto sends its location information in the Notify Payload  with a Notification Type of LOCATION_INFO in the IKE_AUTH request. Notification Type of LOCATION_INFO is defined and configured in both NR Femto and SeGW.

5.	The same as the step 5 in TS 33.320[x] clause A.1.

6.	The SeGW processes the Notify payload of the IKE_AUTH request and verify the location of the NR Femto based on the policy of the operator.

7.	If the location verification is successful in step 6, the SeGW sends IKE_AUTH response to continue the authentication.

NOTE 1:	If the location verification fails,  the following procedure may not be executed based on the operator policy.

8-9.	The same as the steps 8-9 in TS 33.320[x] clause A.1.

* * * End of Changes * * * *