3GPP TSG-SA3 Meeting #120	S3-250636

Athens, Greece, 17 - 21 February 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.545 | CR | - | rev | - | Current version: | 0.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update clause 5.3 for femto location security |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_Femto_Sec |  |  |  |  |  | Date: |  |  | 2025-02-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Principles for normative work related to femto location security are mentioned in TR 33.745 clause 7.3. This CR proposes updates in newly assigned TS 33.545 based on these principles. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updates in normative specifications for 5G NR femto location security. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Normative requirements for 5G NR femto location security will not be included in TS 33.545 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

************* Start of First Change *************

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[X]	3GPP TS 33.320: "Security of Home Node B (HNB) / Home evolved Node B (HeNB)".

********** End of  First Change *************

********** Start of Second Change *************

## 5.3	Location Security

### 5.3.1 General

Operators require assurance of the 5G NR Femto location. to satisfy various security, regulatory and operational requirements. The 5G NR Femto MS and/or 5G NR Femto Gateway (referred to in this section as the “verifying node”) shall perform location verification. One or more types of location information of 5G NR Femto and/or UEs connected to the 5G NR Femto may be optionally stored in the verifying node by operators for location verification. It shall be possible for the verifying node to obtain one or more of the following information which may be used to perform location verification:

- the public IP address of the broadband access device provided by the 5G NR Femto

- the IP address and/or access line location identifier provided by broadband access provider

- information of the neighbouring cells surrounding the 5G NR Femto

- geo-coordinates provided by a GNSS receiver embedded into the 5G NR Femto

- locations of UEs connected to the 5G NR Femto

Different deployment scenarios and 5G NR Femto configurations will influence the availability, accuracy and reliability of these types of location information.

### 5.3.2 IP Address provided by 5G NR Femto

Clause 8.1.2 from TS 33.320 [X] shall be followed for 5G NR Femto.

### 5.3.3 IP Address and/or access line location identifier provided by broadband access provider

Clause 8.1.3 from TS 33.320 [X] shall be followed for 5G NR Femto.

### 5.3.4 Information of the neighbouring cells surrounding the 5G NR Femto

Clause 8.1.4 from TS 33.320 [X] shall be followed for 5G NR Femto. Additionally, neighboring 5G NR Femto cells may also be used for 5G NR Femto location verification.

### 5.3.5 GNSS information provided by 5G NR Femto

Clause 8.1.5 from TS 33.320 [X] shall be followed for 5G NR Femto.

### 5.3.6 Locations of UEs connected to the 5G NR Femto

5G NR Femto cells are expected to provide coverage over a small geographical area. The location of UEs connected to the 5G NR Femto cells can only be within small distances, and this can be used by the verifying nodes to verify the 5G NR Femto location.

### 5.3.7 Requirements

The verifying node shall be capable of requesting one or more of the types of location information listed in clause 5.3.1 of this document. Refer to clause 8.1.6 from TS 33.320 [X] for configuration and location verification procedure (including actions taken post location verification) related requirements applicable to verifying node(s). Also, operator policy aspects can be referred from the same clause 8.1.6 of TS 33.320 [X].

********** End of  Second Change *************