3GPP TSG-SA3 Meeting #120	S3-250652

# Athens, Greece, 17 - 21 February 2025

Source:	Huawei, HiSilicon

Title:	Updates to conclusion for key issue#5 on authentication

Document for:	Approval

Agenda item:	5.9

Spec:	3GPP TR 33.713

Version:	0.6.0

Work Item:	FS_Ambient_IoT_Sec

Comments

This contribution provides the following updates to the conclusion for KI#5 on authentication:

- Clarified that the authentication for the inventory-only procedure is also based on a challenge response and clarified that the authentication data is transported in the inventory messages.

- Clarified that the same mechanism is reused for the inventory and command procedure as well.

- Clarified that for the third party managed AIoT devices, the authentication data are also piggybacked in the inventory messages and transparent to the network.

* * * First Change * * * *

## 7.5	Conclusion on KI #5

1. 	The credential (including device ID and authentication credential) for AIoT device authentication can be owned by an operator or by a third party. The principles for device ID management are concluded in 8.2.1 in TR 23.700-13 [4].

2. 	For operator managed AIoT devices, the following principles are taken as the conclusion for KI#5:

Inventory-only Procedure:

- Authentication (i.e., at least one-way authentication; the 5G network authenticates the AIoT device) shall be supported.

- The authentication procedure is based on a challenge-response mechanism. The challenge from the network is included in the paging request and the response from the device is included in the paging response.

NOTE 1: Details of the authentication procedure (e.g., one-way, parameter(s) used,  etc.)  are to be resolved during normative phase.

Inventory and Command Procedure:

- Mutual authentication between the AIoT device and the 5G network shall be supported.

- The authentication procedure is based on a challenge-response mechanism.

NOTE 2: Details of the mutual authentication procedure and details of the challenge-response mechanism (e.g., parameter(s) used for the 5G network to authenticate the AIoT device and for the AIoT device to authenticate the network) are to be resolved during normative phase.

NOTE 3: Authentication procedures for inventory case and inventory &command case are aligned as much as possible

NOTE 4:	Where to store the AIoT device credentials and related policy information will be based on SA2 and SA3 coordination.

3. 	For third party managed AIoT devices, the operator network relays the authentication data transparently between the AF/credential holder and the device.

* * * End of Changes * * * *