3GPP TSG-SA3 Meeting #120	S3-250662

# Athens, Greece, 17 - 21 February 2025

Source:	Huawei, HiSilicon

Title:	Security aspects on store and forward operation with split MME architecture

Document for:	Approval

Agenda item:	4.19

Spec:	3GPP TS 33.401

Version:	18.2.0

Work Item:	5GSAT_Ph3_SEC

Comments

Based on conclusions agreed in the TR 33.700-29 for KI#1, this proposal proposes the security principles for store and forward operation with a split MME architecture.

* * * First Change * * * *

Annex X (informative):
Security for Store and Forward Satellite operation

X.1	General

Editor’s Note: This clause provides general description for Annex X.

X.2	Security aspects on Split MME architecture

The authentication and communication protection between the UE and the satellite network reuse security procedures as specified in the present document. The NAS security is terminated on the MME-onboard. The latest NAS security context of the UE is available at the MME-onboard which processes the NAS security. The MME-onboard obtains the EPS authentication vectors when the feeder link is available, and stores the EPS authentication vectors when the service link is unavailable.

NOTE 1: The distribution and synchronization of NAS security context between the MME-ground and MME-onboard is out of the scope of 3GPP.

NOTE 2: The security of communications between the proxies on satellite and ground is out of 3GPP scope.

* * * End of Changes * * * *