3GPP TSG-SA3 Meeting #120	S3-250679

# Athens, Greece, 17 - 21 February 2025

Source:	Huawei, HiSilicon

Title:	Add Security flow of Multi-hop UE-to-Network Relay discovery procedure to ProSe draftCR

Document for:	Approval

Agenda item:	4.15

Spec:	3GPP TS 33.503

Version:	18.5.0

Work Item:	5G_ProSe_Sec_Ph3

Comments

Accept the following changes to add security flow of multi-hop UE-to-Network discovery procedures to the ProSe Ph3 draft CR.

* * * First Change * * * *

6.1.3.X.3	Security flows for 5G ProSe Multi-hop UE-to-Network Relay Discovery

The security flows of 5G ProSe Multi-hop UE-to-Network Relay discovery are based on the security procedures of 5G ProSe UE-to-Network Relay discovery as specified in clause 6.1.3.2, including both Model A and Model B discovery.

The 5G ProSe Remote UE, Intermediate UE-to-Network Relay, and 5G ProSe UE-to-Network Relay are provisioned with the discovery security materials associated with an RSC from the 5G PKMF/5G DDNMF of 5G ProSe Remote UE, Intermediate UE-to-Network Relay, and 5G ProSe UE-to-Network Relay’s HPLMN respectively, based on the procedure specified in clause 6.1.3.2. The provisioned discovery security materials contain a Discovery User Integrity Key (DUIK) for the integrity protection of discovery messages.

* * * Second Change * * * *

6.1.3.X.3.1	Security procedure for 5G ProSe Multi-hop UE-to-Network Relay Discovery with Model A

In the Model A multi-hop UE-to-Network Relay Discovery scenario as specified in clause 6.3.2.5.2 of TS 23.304 [2], the Intermediate UE-to-Network Relay and the UE-to-Network Relay can perform the role of the Announcing UE to send the Relay Discovery Announcement message, while the Remote UE and the Intermediate UE-to-Network Relay can perform the role of the Monitoring UE to receipt the Relay Discovery Announcement message.

The monitoring Remote UE is provisioned with the RSC and the discovery security materials based on the security materials provisioning mechanism for UE-to-Network Relay Discovery as specified in clause 6.1.3.2.2.1 of the present document. The provisioned discovery security materials contain a Discovery User Integrity Key (DUIK) for the integrity protection of discovery messages.

The monitoring Intermediate UE-to-Network Relay, announcing Intermediate UE-to-Network Relay and the announcing UE-to-Network Relay are provisioned with the RSC and discovery security materials associated with their own HPLMN ID based on the security materials provisioning mechanism for UE-to-Network Relay Discovery as specified in clause 6.1.3.2.2.1 of the present document. The provisioned discovery security materials contain a Discovery User Integrity Key (DUIK) for the integrity protection of discovery messages.

When sending the Relay Discovery Announcement message, the Announcing UE shall protect the message as specified in clause 6.1.3.2.3 of the present document, using the provisioned discovery security materials. The Monitoring UE processes the received discovery message as specified in step 12 of clause 6.1.3.2.2.1 of present document, using the provisioned discovery security materials. In the case of the Intermediate UE-to-Network Relay acts as monitoring UE and after successfully processing the discovery message using the provisioned discovery security materials, the Intermediate UE-to-Network Relay updates the path information (e.g., hop count, Relay Info) and protects the updated message as specified in clause 6.1.3.2.3 of the present document using the same security materials.

* * * Third Change * * * *

6.1.3.X.3.2	Security procedure for 5G ProSe Multi-hop UE-to-Network Relay Discovery with Model B

In the Model B multi-hop UE-to-Network Relay Discovery scenario as specified in clause 6.3.2.5.3 of TS 23.304 [2], the Intermediate UE-to-Network Relay and the UE-to-Network Relay can perform the role of the Discoveree UE to receive the Discovery Solicitation and reply the Discovery Response, while the Remote UE and the Intermediate UE-to-Network Relay can perform the role of the Discoverer UE to send the Discovery Solicitation and receive the Discovery Response.

The Remote UE is provisioned with the RSC and the discovery security materials based on the security materials provisioning mechanism for UE-to-Network Relay Discovery as specified in clause 6.1.3.2.2.2 of the present document. The provisioned discovery security materials contain a Discovery User Integrity Key (DUIK) for the integrity protection of discovery messages.

The Intermediate UE-to-Network Relay and the announcing UE-to-Network Relay are provisioned with the RSC and discovery security materials associated with their own HPLMN ID based on the security materials provisioning mechanism for UE-to-Network Relay Discovery as specified in clause 6.1.3.2.2.2 of the present document. The provisioned discovery security materials contain a Discovery User Integrity Key (DUIK) for the integrity protection of discovery messages.

When sending the Discovery Solicitation message, the Discoverer UE shall protect the message as specified in clause 6.1.3.2.3 of the present document, using the provisioned discovery security materials. The Discoveree UE processes the received Discovery Solicitation as specified in step 13 of clause 6.1.3.2.2.2 of present document, using the provisioned discovery security materials.

When sending the Discovery Response message, the Discoveree UE shall protect the message as specified in clause 6.1.3.2.3 of the present document, using the provisioned discovery security materials. The Discoverer UE processes the received Discovery Response as specified in step 14 of clause 6.1.3.2.2.2 of present document, using the provisioned discovery security materials.

In the case of the Intermediate UE-to-Network Relay acts as Discoceree UE and after successfully processing the Discovery Solicitation message using the provisioned discovery security materials , the Intermediate UE-to-Network Relay updates the path information (e.g., hop count, Relay Info) and protects the updated message using the same security materials as specified in clause 6.1.3.2.3 of the present document.

* * * End of Changes * * * *