3GPP TSG-SA3 Meeting #120	S3-250691

Athens, Greece, 17 - 21 February 2025

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.116 | CR | 0005 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Added missing references |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | SCAS_5G_Maint |  |  |  |  |  | Date: |  |  | 2025-02-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Requirement Reference and Threat References are missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add content to Requirement Reference and Threat References |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.2.1, 4.2.2.2, 4.2.2.3, 4.2.2.4, 4.2.2.5, 42.2.23.1, 4.2.2.3.2, 4.2.2.3.3, 4.2.2.3.4, 4.2.2.4.1, 4.2.2.4.2, 4.2.2.5.1, 4.2.2.5.2, 4.2.2.5.3, 4.2.2.6.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**************** START of CHANG****************

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 41.001: "GSM Specification set".

[3]	3GPP TR 33.117: "Catalogue of General Security Assurance Requirements".

[4]	3GPP TR 33.916: "Security assurance scheme for 3GPP network products for 3GPP network product classes".

[5]	3GPP TS 33.401: "3GPP System Architecture Evolution (SAE); Security architecture".

[6]	void.

[7]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[8]	3GPP TS 33.102: "3G security; Security architecture".

[X]	3GPP TR 33.926: "Security Assurance Specification (SCAS) threats and critical assets in 3GPP network product classes".

**************** End of 1st CHANG****************

**************** START of 2nd CHANG****************

4.2.2.2.1 	Access with 2G SIM forbidden

Requirement Name: 2G SIM access forbidden

Requirement Reference: TS 33.401[5], clause 6.1.1

Requirement Description: "Access to E-UTRAN with a 2G SIM or a SIM application on a UICC shall not be granted." as specified in TS 33.401[5], clause 6.1.1.

Threat References: TR 33.926 [X], clause A.2.2.1 Access to 2G.

Test Case:

Purpose:

Verify that access to EPS with a 2G SIM is not possible.

Pre-Conditions:

Test environment with HSS. HSS may be simulated.

Execution Steps

Include 2G authentication vector in authentication data response from HSS.

Expected Results:

MME rejects UE authentication when receiving 2G authentication vector from HSS.

NOTE: 	When both MME and HSS function correctly 2G authentication vector are never included in authentication data response from HSS to MME.

**************** END of 2nd CHANG****************

**************** START of 3rd CHANG****************

4.2.2.2.2 	Re-synchronization

Requirement Name: Inclusion of RAND, AUTS

Requirement Reference: TS 33.401[5], clause 6.1.2

Requirement Description: "In the case of a synchronization failure, the MME shall also include RAND and AUTS." as specified in TS 33.401[5], clause 6.1.2.

Threat References: TR 33.926 [X], clause A.2.2.2 Resynchronization.

Test Case: Purpose:

Verify that Re-synchronization procedure works correctly.

Pre-Conditions:

Test environment with UE and HSS. UE and HSS may be simulated.

Execution Steps

The MME receives an AUTHENTICATION FAILURE message, with the EMM cause #21 "synch failure" and a re-synchronization token AUTS.

Expected Results:

The MME includes the stored RAND and the received AUTS in the authentication data request to the HSS.

NOTE: 	When RAND and AUTS are not included in the authentication data request to the HSS then the HSS will return a new authentication vector (AV) based on its current value of the sequence number SQNHE (cf. TS 33.102, clause 6.3.5) A new authentication procedure between MME and UE using this new AV will be successful just the same if the cause of the synchronisation failure was the sending of a "stale" challenge, i.e. one that the UE had seen before or deemed to be too old. But if the cause of the synchronisation failure was a problem with the sequence number SQNHE in the HSS (which should be very rare), and the RAND and AUTS are not included in the authentication data request to the HSS, then an update of SQNHE based on AUTS will not occur in the HSS, and the new authentication procedure between MME and UE using the new AV will fail again. This can be considered a security-relevant failure case as it may lead to a subscriber being shut out from the system permanently.

**************** END of 3rd CHANG****************

**************** START of 4th CHANG****************

4.2.2.2.3 	Integrity check of Attach message

Requirement Name: Integrity check of Attach message

Requirement Reference: TS 33.401[5], clause 6.1.4

Requirement Description: "If the user cannot be identified or the integrity check fails, then the MME shall send a response indicating that the user identity cannot be retrieved." as specified in TS 33.401[5], clause 6.1.4.

Threat References: TR 33.926 [X], clause A.2.2.3 Failed Integrity check of Attach message.

Test Case:

Purpose:

Verify that secure user identification by means of integrity check of Attach request works correctly.

Pre-Conditions:

Test environment with new and old MME. New MME may be simulated.

Execution Steps

The old MME receives an Identification Request message from the new MME with incorrect integrity protection.

Expected Results:

The old MME sends a response indicating that the user identity cannot be retrieved.

**************** END of 4th CHANG****************

**************** START of 5th CHANG****************

4.2.2.2.4 	Not forwarding EPS authentication data to SGSN

Requirement Name: Not forwarding EPS authentication data to SGSN

Requirement Reference: TS 33.401[5], clause 6.1.4

Requirement Description: "EPS authentication data shall not be forwarded from an MME towards an SGSN." as specified in TS 33.401[5], clause 6.1.4.

Threat References: TR 33.926 [X], clause A.2.2.4 Forwarding EPS authentication data to SGSN.

Test Case:

Purpose:

Verify that EPS authentication data remains in the EPC.

Pre-Conditions:

Test environment with MME and SGSN. SGSN may be simulated.

Execution Steps

The MME receives an Identification Request message from the SGSN.

Expected Results:

The response to the SGSN does not include EPS authentication data.

**************** END of 5th CHANG****************

**************** START of 6th CHANG****************

4.2.2.2.5 	Not forwarding unused EPS authentication data between different security domains

Requirement Name: Not forwarding unused EPS authentication between different security domains

Requirement Reference:TS 33.401[5], clause 6.1.5

Requirement Description: "Unused EPS authentication vectors, or non-current EPS security contexts, shall not be distributed between MMEs belonging to different serving domains (PLMNs)." as specified in TS 33.401[5], clause 6.1.5.

Threat References: TR 33.926 [X], clause A.2.2.5 Forwarding unused EPS authentication data between different security domains.

Test Case:

Purpose:

Verify that unused EPS authentication data remains in the same serving domain.

Pre-Conditions:

Test environment with old and new MME in different serving domains. New MME may be simulated.

Execution Steps

The old MME receives an Identification Request message from the new MME.

Expected Results:

The response to the new MME does not include unused EPS authentication data.

**************** END of 6th CHANG****************

**************** START of 7th CHANG****************

4.2.2.3.1 	Bidding down prevention

Requirement Name: Bidding down prevention

Requirement Reference: TS 33.401[5], clause 7.2

Requirement Description: "The SECURITY MODE COMMAND shall include the replayed security capabilities of the UE." as specified in TS 33.401[5], clause 7.2.

Threat References: TR 33.926 [X], A.2.3.1 Bidding Down.

Security Objective References: TBA

Test Case:

Purpose:

Verify that bidding down by eliminating certain UE capabilities on the interface from UE to MME is not possible.

Pre-Conditions:

Test environment with UE. UE may be simulated.

Execution Steps

Attach request message includes security capabilities of the UE.

Expected Results:

MME includes the same security capabilities of the UE in the SECURITY MODE COMMAND message.

**************** END of 7th CHANG****************

**************** START of 8th CHANG****************

4.2.2.3.2 	NAS integrity algorithm selection and use

Requirement Name: NAS integrity algorithm selection

Requirement Reference: TS 33.401[5], clause 7.2.4.3.1

Requirement Description: "The MME shall protect the SECURITY MODE COMMAND message with the integrity algorithm, which has the highest priority according to the ordered lists." as specified in TS 33.401[5], clause 7.2.4.3.1.

NOTE: 	The text in TS 33.401, clause 7.2.4.3.1 is somewhat incomplete. It should properly read: "…which has the highest priority according to the ordered lists and is contained in the UE EPS security capabilities."

Threat References: TR 33.926 [X], A.2.3.2 NAS integrity selection and use

Test Case:

Purpose:

Verify that NAS integrity protection algorithm is selected and applied correctly.

Pre-Conditions:

Test environment with UE. UE may be simulated.

Execution Steps

The MME sends the SECURITY MODE COMMAND message. The UE replies with the SECURITY MODE COMPLETE message.

Expected Results:

1.	The MME has selected the integrity algorithm which has the highest priority according to the ordered lists and is contained in the UE EPS security capabilities. The MME checks the message authentication code on the SECURITY MODE COMPLETE message.

2.	The MAC in the SECURITY MODE COMPLETE is verified, and the NAS integrity protection algorithm is selected and applied correctly.

**************** END of 8th CHANG****************

**************** START of 9th CHANG****************

4.2.2.3.3 	NAS NULL integrity protection

Requirement Name: NAS NULL integrity protection

Requirement Reference: TS 33.401[5], clause 5.1.4.1

Requirement Description: "EIA0 shall only be used for unauthenticated emergency calls." as specified in TS 33.401[5], clause 5.1.4.1."

Threat References: TR 33.926 [X], A.2.3.3 NAS NULL integrity protection

Test Case:

Purpose:

Verify that NAS NULL integrity protection algorithm is used correctly.

Pre-Conditions:

Test environment with UE. UE may be simulated.

Execution Steps

The MME sends the SECURITY MODE COMMAND message after successful UE authentication.

Expected Results:

The selected integrity algorithm is different from EIA0.

**************** END of 9th CHANG****************

**************** START of 10th CHANG****************

4.2.2.3.4 	NAS confidentiality protection

Requirement Name: NAS confidentiality protection

Requirement Reference: TS 33.401[5], clause 7.2.4.3.1

Requirement Description: "The UE…sends the NAS security mode complete message to MME ciphered and integrity protected." as specified in TS 33.401[5], clause 7.2.4.3.1.

Threat References: TR 33.926 [X], A.2.3.4 NAS confidentiality protection

Test Case:

Purpose:

Verify that NAS confidentiality protection algorithm is applied correctly.

Pre-Conditions:

Test environment with UE. UE may be simulated.

Execution Steps

The MME receives the SECURITY MODE COMPLETE message without confidentiality protection.

Expected Results:

If a confidentiality algorithm different from EEA0 was selected the MME rejects the message.

**************** END of 10th CHANG****************

**************** START of 11th CHANG****************

4.2.2.4.1 	Bidding down prevention in X2-handovers

Requirement Name: Bidding down prevention in X2-handovers

Requirement Reference: TS 33.401[5], clause 7.2.4.2.2

Requirement Description: "The MME shall verify that the UE EPS security capabilities received from the eNB are the same as the UE EPS security capabilities that the MME has stored." as specified in TS 33.401[5], clause 7.2.4.2.2."

Threat References: TR 33.926 [X], A.2.4.1 Bidding down on X2-Handover

Test Case:

Purpose:

Verify that bidding down is prevented in X2-handovers.

Pre-Conditions:

Test environment with (target) eNB. eNB may be simulated.

The MME is configured to log the event of a UE EPS security capability mismatch.

Execution Steps

The MME receives the path-switch message with the UE EPS security capabilities different from the ones stored in the MME for that UE.

Expected Results:

The MME logs the event.

**************** END of 11th CHANG****************

**************** START of 12th CHANG****************

4.2.2.4.2 	NAS integrity protection algorithm selection in MME change

Requirement Name: NAS integrity protection algorithm selection in MME change

Requirement Reference: TS 33.401[5], clause 7.2.4.3.2

Requirement Description: "In case there is change of MMEs and algorithms to be used for NAS, the target MME shall initiate a NAS security mode command procedure and include the chosen algorithms and the UE security capabilities (to detect modification of the UE security capabilities by an attacker) in the message to the UE (see clause 7.2.4.4). The MME shall select the NAS algorithms which have the highest priority according to the ordered lists (see clause 7.2.4.3.1)." as specified in TS 33.401[5], clause 7.2.4.3.2.

Threat References: TR 33.926 [X], A.2.4.2 NAS integrity protection algorithm selection in MME change

Test Case:

Purpose:

Verify that NAS integrity protection algorithm is selected correctly.

Pre-Conditions:

Test environment with source and target MME. Source MME may be simulated.

Execution Steps

The target MME receives the UE EPS security capabilities and the NAS algorithms used by the source MME from the source MME over the S10 interface. The target MME selects the NAS algorithms which have the highest priority according to the ordered lists. The lists are assumed such that the algorithms selected by the target MME are different from the ones received from the source MME.

Expected Results:

The target MME initiates a NAS security mode command procedure and include the chosen algorithms and the UE security capabilities.

**************** END of 12th CHANG****************

**************** START of 13th CHANG****************

4.2.2.5.1 	No access with 2G SIM via idle mode mobility

Requirement Name: Idle mode mobility into E-UTRAN forbidden for GSM subscribers

Requirement Reference: TS 33.401[5], clause 9.1.2

Requirement Description: "In case the MM context in the Context Response/SGSN Context Response indicates GSM security mode, the MME shall abort the procedure." as specified in TS 33.401[5], clause 9.1.2.

Threat References: TR 33.926 [X], A.2.5.1 2G SIM access via idle mode mobility

Test Case:

Purpose:

Verify that 2G subscribers cannot obtain service in EPS via idle mode mobility.

Pre-Conditions:

Test environment with source SGSN and target MME. Source SGSN may be simulated.

Execution Steps: The target MME receives the MM context in the Context Response indicating GSM security mode.

Expected Results: The MME aborts the procedure by acknowledging the Context Response from the SGSN with an appropriate failure cause.

**************** END of 13th CHANG****************

**************** START of 14th CHANG****************

4.2.2.5.2 	No access with 2G SIM via handover

Requirement Name: Handover into E-UTRAN forbidden for GSM subscribers

Requirement Reference: TS 33.401[5], clause 9.2.2

Requirement Description: "In case the MM context in the Forward relocation request message indicates GSM security mode (i.e. it contains a Kc), the MME shall abort the non-emergency call procedure." as specified in TS 33.401[5], clause 9.2.2.

Threat References: TR 33.926 [X], A.2.5.3 2G SIM access via SRVCC

Test Case:

Purpose:

Verify that GSM subscribers cannot obtain service in EPS via handovers.

Pre-Conditions:

Test environment with source SGSN and target MME. Source SGSN may be simulated.

Execution Steps: The target MME receives the MM context in the Forward Location Request message indicating GSM security mode.

Expected Results: The MME aborts the procedure by responding to the Forward Relocation Request from the SGSN with an appropriate failure cause.

**************** END of 14th CHANG****************

**************** START of 15th CHANG****************

4.2.2.5.3 	No access with 2G SIM via SRVCC

Requirement Name: SRVCC into E-UTRAN forbidden for GSM subscribers

Requirement Reference: TS 33.401[5], clause 14.3.1

Requirement Description: "If the MME receives a GPRS Kc' from the source MSC server enhanced for SRVCC in the CS to PS HO request, the MME shall reject the request." as specified in TS 33.401[5], clause 14.3.1.

Threat References: TR 33.926 [X], A.2.6	Threats related to release of non-emergency bearer

Test Case:

Purpose:

Verify that GSM subscribers cannot obtain service in EPS via SRVCC into E-UTRAN.

Pre-Conditions:

Test environment with source MSC server and target MME. Source MSC server may be simulated.

Execution Steps

The target MME receives the GPRS Kc' and the CKSN'PS in the CS to PS handover request.

Expected Results:

The MME rejects the request.

**************** END of 15th CHANG****************

**************** START of 16th CHANG****************

4.2.2.6.1 	Authentication failure for emergency bearers

NOTE: 	The use of NULL integrity is addressed in clause D.3.2.3.3.

Requirement Name: Emergency bearer establishment when authentication fails

Requirement Reference: TS 33.401 [5], clause 15.1.

Requirement Description: "The MME or UE shall always release any established non-emergency bearers, when the authentication fails in the UE or in the MME." as specified in TS 33.401[5], clause 15.1.

Threat References: TBA

Test Case:

Purpose:

Ensure that the MME enforces that only emergency bearers can be used without successful authentication.

Pre-Conditions:

Test environment with MME and UE. UE may be simulated. The serving network policy allows unauthenticated IMS Emergency Sessions.

Execution Steps

The UE sends the initial attach request for EPS emergency bearer services, then the MME initiates an authentication, which fails. The UE attached for EPS emergency bearer services sends the PDN Connectivity request for EPS non-emergency bearer services.

Expected Results:

The MME allows to continue the set up of the emergency bearer, and will reject the PDN Connectivity request for EPS non-emergency bearer services.

**************** END of CHANG****************