3GPP TSG-SA3 Meeting #120	S3-250692

Athens, Greece, 17 - 21 February 2025

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.250 | CR | 0006 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding missing references |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | SCAS_5G_Maint |  |  |  |  |  | Date: |  |  | 2025-02-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Requirement Reference and Threat References are missing.. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Corrected missing references |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.2.5, 4.2.2.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**************** START of CHANG****************

4.2.2.5	Mobility binding

Requirement Name: MN-HA authentication extension validation for mobility binding during trusted non-3GPP access

Requirement Reference: TS 33.402 [11], clause 9.2.1.1

Requirement Description: "The PDN-GW shall validate the MN-HA authentication extension" as specified in TS 33.402 [11], clause 9.2.1.1.

Threat References: TBA

Test Case:

Test Name: TC_PGW_MIP-AUTH_Non-3GPP

Purpose: To test whether the PGW validates the MN-HA authentication extension correctly.

Pre-Condition:

The UE and PGW are connected in the test environment. UE is simulated.

The tester has access to the S6b interface between the 3GPP AAA Server and the PDN GW.

The tester has access to the MIPv4 FACoA based S2a interface between the PGW and UE.

Execution Steps:

The PGW (home agent for the UE) is active in a 3GPP access network.

The tester captures packets over S6b and S2a interfaces using any packet analyser.

The tester filters the MN-HA Key transported in the authentication and authorization information from the 3GPP AAA server to PGW over S6b interface.

The UE sends a Registration Request message to PGW via the trusted non-3GPP IP access network.

The tester filters the Registration Request sent by UE to PGW and Registration Reply sent by PGW to UE over the S2a interface.

The tester uses the SPI value in Registration Request to identify the MN-HA key to compute the Authenticator value of the authentication extension.

The tester verifies that the computed Authenticator value is same as the Authenticator value in the Registration Request message.

The tester also checks the Reply Code in the Registration Reply message to verify the correctness of the validation at PGW.

Expected Results:

The Reply code is ‘0’ in the Registration Reply message.

Expected format of evidence:

Evidence suitable for the interface, e.g. Screenshot contains the operation results.

**************** END of 1st CHANG****************

**************** START of 2nd  CHANG****************

4.2.2.6	Inactive emergency PDN connection release

Requirement Name: Emergency PDN connection release

Requirement Reference: TS 23.401 [6], clause 5.4.4.1

Requirement Description: “PGW shall initiate the deactivation of all bearers of the emergency PDN connection when it is inactive (i.e. not transferring any packets) for a configured period of time.” as specified in TS 23.401 [6], clause 5.4.4.1.

Threat References: TR 33.926[10], clause B.2.4.1 Inactive Emergency PDN Connection Release

Test Case:

Test Name: TC_PGW_EMRGENCY_CONNECTION-RELEASE

Purpose: To verify whether the PGW releases all emergency PDN connections which are inactive for a configured inactive time to prevent resource exhaustion.

Pre-Condition:

The UE, PGW and S-GW network products are connected in the test environment.

UE and S-GW may be simulated.

The tester has access to the PGW configuration file.

The tester has access to the GTP-based S5/S8 interface.

Execution Steps:

1)	The tester checks the PGW configuration file to find the PDN connection’s inactive timeout value.

2)	The tester initiates an emergency attach procedure.

3)	The tester captures the packet over S5/S8 interface between PGW and S-GW using any packet analyser.

4)	The tester filters the PDN CONNECTIVITY REQUEST message with request type "emergency" during the attach procedure.

5)	The tester filters the DELETE BEARER REQUEST messages (Procedure Transaction Identifier, EPS Bearer Identity, Causes) sent from PGW to Serving GW.

6)	The tester also filters the corresponding DELETE BEARER RESPONSE messages (EPS Bearer Identity, User Location Information) sent from Serving GW to PGW.

7)	The tester verifies whether the Cause value of the DELETE BEARER REQUEST message is "PDN connection inactivity timer expires". If yes proceed, otherwise go to step 5.

8)	To confirm the emergency bearer release, the tester compares whether the EPS Bearer identity of the UE is the same in all three messages (PDN CONNECTIVITY REQUEST, DELETE BEARER REQUEST and DELETE BEARER RESPONSE), otherwise it may be concluded that the inactive emergency PDN connection is not released even after the configured timeout.

Expected Results:

The PGW releases the inactive emergency bearers according to the configured timeout value.

Expected format of evidence:

Evidence suitable for the interface, e.g. Screenshot contains the operation results.

**************** END of CHANG****************