3GPP TSG-SA3 Meeting #120	S3-250728

Athens, Greece, 17 - 21 February 2025

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.516 | CR | draftCR | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add test case that verifies if the AUSF processes RES* failures correctly |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | BSI, Montsecure |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | SCAS_5G_Maint |  |  |  |  |  | Date: |  |  | 2025-02-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The AMF has a test case named TC_RES_STAR_VERIFICATION_FAILURE that verifies whether RES* verification failure is handled correctly. Especially in a roaming scenario, this verification is done in the AUSF network function for the home network, since the AMF/SEAF is part of the visiting network in such case. If verification in the AUSF is not correctly implemented, this might have security implications, as seen in some open source implementations. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added a test case TC_RES_STAR_VERIFICATION_FAILURE_AUSF that tests whether RES* verification is correctly implemented in the AUSF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Security key K_SEAF might be leaked if the AUSF network function does not implement RES* verification correctly (more information in TR 33.926). |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.2.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TR 33.926 CR 0102 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

********** START OF 1st CHANGE **********

##### 4.2.2.X	RES* verification failure handling

Requirement Name: No linkability between SUCI and SUPI in AUSF confirmation

Requirement Reference: TS 33.501, clause 6.1.3.2.0 and TS 29.509, clause 5.2.2.2.2.

Requirement Description:

As specified in TS 33.501, clause 6.1.3.2.0, the AUSF proceeds with step 11 in Figure 6.1.3.2-1 after receiving a Nausf_UEAuthentication_Authenticate request message from the SEAF. The AUSF should behave in the following way:

-	If the 5G Authentication Vector is valid, the AUSF compares RES* with XRES*:

-	If they are equal, the AUSF should accept the authentication.

-	If they are not equal, the AUSF should deny the authentication.

Additionally, the AUSF should always inform the UDM about the authentication result.

If a failure occurred while authenticating the UE in the SEAF, the SEAF will send a Nausf_UEAuthentication_Authenticate request message with a RES* value set to null to indicate to the AUSF that a failure has occurred as specified in TS 29.509, clause 5.2.2.2.2.

Threat Reference: TR 33.926, clause F.2.2.X, RES* verification failure

Test case:

Test Name: TC_RES_STAR_VERIFICATION_FAILURE_AUSF

Purpose:

Test case 1:	Verify that the AUSF correctly handles RES* verification when the HRES* value is the same as the RES* value.

Test case 2:	Verify that the AUSF correctly handles RES* verification when the HRES* value is set an incorrect value (value not equal to RES*).

Test case 3:	Verify that the AUSF correctly handles RES* verification when the HRES* value is set to null.

Pre-Conditions:

-	AUSF network product is connected in simulated/real network environment including a UDM and AMF. The UDM and AMF may be simulated.

-	The The tester has access to the user credentials.

-	The SUPI should be registered in the UDR.

Execution Steps:

Test case 1 (RES* equals HRES*):

1.	The tester triggers an Nausf_UEAuthentication request message sent from AMF to AUSF over the N12 interface containing the SUCI derived from the SUPI.

2.	The tester captures the Nausf_UEAuthentication response message sent from AUSF to AMF over the N12 interface containing HXRES*, AUTN and RAND values.

3.	The tester computes the HRES* value with the RAND value.

4.	The tester triggers an Nausf_UEAuthentication_Authenticate request message sent from AMF to AUSF over the N12 interface containing the computed HRES* value.

5.	The tester captures the Nausf_UEAuthentication_Authenticate response message sent from AUSF to AMF over the N12 interface.

Test case 2 (RES* incorrect and not null):

1.	The tester triggers an Nausf_UEAuthentication request message sent from AMF to AUSF over the N12 interface containing the SUCI derived from the SUPI.

2.	The tester captures the Nausf_UEAuthentication response message sent from AUSF to AMF over the N12 interface containing HXRES*, AUTN and RAND values.

3.	The tester computes the HRES* value with the RAND value. The tester then alters the computed HRES* value to make it invalid. The HRES* value should not be set to a vector filled with 0 bytes.

4.	The tester triggers an Nausf_UEAuthentication_Authenticate request message sent from AMF to AUSF over the N12 interface containing the altered HRES* value.

5.	The tester captures the Nausf_UEAuthentication_Authenticate response message sent from AUSF to AMF over the N12 interface.

Test case 3 (HRES* set to null):

1.	The tester triggers an Nausf_UEAuthentication request message sent from AMF to AUSF over the N12 interface containing the SUCI derived from the SUPI.

2.	The tester captures the Nausf_UEAuthentication response message sent from AUSF to AMF over the N12 interface containing HXRES*, AUTN and RAND values.

3.	The tester triggers an Nausf_UEAuthentication_Authenticate request message sent from AMF to AUSF over the N12 interface containing null as the HRES* value.

4.	The tester captures the Nausf_UEAuthentication_Authenticate response message sent from AUSF to AMF over the N12 interface.

Expected Results:

For test case 1, the AUSF responds with a successful authentication response. Additionally, the AUSF notifies the UDM of the successful authentication over the N13 interface.

For test cases 2 and 3, the AUSF responds with an unsuccessful authentication response. Additionally, the AUSF notifies the UDM of the unsuccessful authentication over the N13 interface.

Expected format of evidence:

Evidence suitable for the interface, e.g., evidence can be presented in the form of log messages or a packet trace. A packet trace should at least contain the messages sent between the AMF and AUSF on the N12 interface and the messages sent between the AUSF and UDM on the N13 interface.

********** END OF CHANGE **********