3GPP TSG-SA3 Meeting #120	S3-250733

Athens, Greece, 17 - 21 February 2025

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.256 | CR | 0052 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UUAA in 5GS updates |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | UAS_Ph3_Sec |  |  |  |  |  | Date: |  |  | 2025-02-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Updates to UUAA. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updates to UUAA. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.1.1, 5.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** START OF CHANGES ***

## 5.2	UUAA

### 5.2.1	UUAA in 5GS

#### 5.2.1.1	General

The UAV USS authentication and authorization (UUAA) is the procedure to ensure that the UAV can be authenticated and authorized by a USS before the connectivity for UAS services is enabled. This clause specifies the relationship between primary authentication (as described in clause 6.1 in TS 33.501 [2]) and UUAA. An UAV is allowed to perform UUAA with the USS/UTM only after the UAV (UE) has completed successfully primary authentication.

It may be triggered by the AMF when UAV is registering with 5GS or triggered by the SMF during the PDU session establishment procedure. The UUAA procedure may also be triggered by a USS for re-authentication if the USS had authenticated the UAV. Network support for UUAA during registration is optional while it is mandatory during the PDU Session establishment. UE Support for UUAA during registration and during the PDU Session establishment is mandatory.

The AMF or SMF triggers the UUAA procedure if the UAV has an Aerial UE subscription and the UAV requests access to UAS services by providing the CAA-Level UAV ID of the UAV in the Registration Request or PDU Session Establishment Request.

The UUAA is performed between the UAV and the USS. The UAV is authenticated based on the CAA-Level UAV ID and credentials associated to the CAA-Level UAV ID. The authentication messages are included in a transparent container and conveyed between the UAV and the USS via a 3GPP UAS NF.

NOTE:	The provision of CAA-Level UAV ID, credentials, and the actual authentication methods and information that needs to be sent to perform the UUAA are out of scope of the 3GPP specifications.

On successful completion of a UUAA, the USS can send UAS security information in the UUAA Authorization Payload to the UAV. The contents of that security information are out of scope of the 3GPP specifications.

One or more USS(s) may be present in a specific region and may manage UAVs over one or more 3GPP networks. This feature introduces support for multiple USS serving different geographical areas corresponding to the UAV flight path, i.e. changeover of the UAV from one USS to another USS.

The 3GPP Network subscription for the UAV is not assumed to contain any information about the USS.

The USS address(es), if known to the UAV, is/are configured in the UAV via mechanisms outside the scope of 3GPP. Multiple USS addresses may be configured/provisioned to UAV in case that multiple USSs are deployed in a geographic area and may serve the UAV flight path.

The UAS NF requests information from relevant NFs for determining whether UAV has left or is about to leave the service area of one USS and moves into area of another USS.

A USS requests the core network to provide the assistance information for the serving USS, to inform the core network about the flight trajectory assigned and time schedule for the UAV/UE, and to receive notifications when the UAV/UE moves out/in the geographical area served by a USS triggering and completing the changeover.

The UUAA procedure at registration in 5G is described in the clause 5.2.1.2 and the UUAA procedure during PDU session establishment procedure is described in the clause 5.2.1.3.

At any time after the initial registration, the USS or the AMF (when the networking supports UUAA during registration) may initiate the Re-authentication procedure for the UAV. The AMF initiated Re-authentication procedure is described in the clause 5.2.1.2, whereas the USS initiated Re-authentication procedure is described in the clause 5.1.2.4.

Figure 5.2.1.1-1 provides an example of how UUAA fits into the 5GS procedures. The complete description of this flow is given in TS 23.256 [3].

Figure 5.2.1.1-1: UUAA in 5GS

1.	The UE sends a Registration Request message to the AMF. The UE may provide a CAA-Level UAV ID, and optionally a USS address/IP address, to indicate the request is registering for UAS services. In case the CAA-Level UAV ID and/or USS address/IP address is configured not to be sent in plain text, e.g., the USS address or an IP address not to be exposed in public, the CAA-Level UAV ID, and USS/IP address if available, shall be sent after the NAS security is established.

2.	AMF completes security set up including primary authentication as needed.

3.	After successful Primary authentication, AMF determines whether UUAA is required for the UE. UUAA shall only be triggered if the UE has provided a CAA-Level UAV ID and has a valid Aerial UE subscription. AMF may skip UUAA if the UE has completed UUAA successfully before and the UE UUAA is current, i.e., the UE's authentication and authorization has not been revoked after a previous successful UUAA.

4a. AMF shall return a Registration Accept message to the UE and indicate that UUAA is pending.

4b.	UE may send a Registration Complete message to acknowledge the AMF.

5. AMF triggers the UUAA procedure if determined needed in step 3 as described in clause 5.2.1.2.

The following procedure is for UUAA during PDU session establishment:

6. The UE sends a PDU Session Establishment Request message to the SMF including a CAA-Level UAV ID to indicate the request is for UAS services.

7. The SMF determines whether UUAA is required for the UE. UUAA shall only be triggered if the UE has provided a CAA-Level UAV ID and has a valid Aerial UE subscription. SMF may skip UUAA, the UE has completed UUAA successfully with the same USS/DN before, i.e., at registration as in step 5 or in previous PDU Session Establishment procedures and a successful UUAA result is available.

8. The SMF triggers the UUAA procedure if determined needed at step 7 as described in clause 5.2.1.3.

*** NEXT CHANGES ***

### 5.X	Procedure for USS changeover during a UAV flight

The steps 1-21 of the procedure of USS changeover as specified in the clause 5.13.2 of the TS 23.256 [3] are amended as follows (only affected steps are described):

Step 5:		The NEF shall check whether its UUAA context contains the UUAA results for candidate USSs.

Step 6:		The NEF shall perform the flight assistance procedure as in the clause 5.13.2 of the TS 23.256 [3].

Step 8:		In the response message, the NEF shall include info about whether the candidate USSs require UUAA.

Step 9:		The serving USS (e.g., USS1) may choose the target USS considering the UUAA result of the candidate USSs stored in the UUAA context and may indicate whether the UUAA is required or not for the selected target USS.

Step 20:	If the serving USS selects a target USS (e.g., USS2) for which the USS requests/requires UUAA, the serving USS1 shall inform the UAV or the target USS to perform UUAA as in the clause 5.13.2 of the TS 23.256 [3].

The UAV shall trigger the UUAA as specified in the clause 5.2.1.1 with the target USS address received from the serving USS.

Alternatively, the selected target USS2 shall send an authentication request to the NEF and the NEF shall send a UUAA request to the UAV as specified in the clause 5.2.1.4.

NOTE:	The USS2 needs to establish a security link between the USS2 and the NEF before performing the UUAA procedure.

The NEF shall store the USS2’s UUAA results in the UUAA context as specified in the clause 5.2.1.3 if successful.

*** END OF CHANGES ***