3GPP TSG-SA3 Meeting #120	S3-250770

# Athens, Greece, 17 - 21 February 2025

Source:	Samsung

Title:	Updates to conclusion for key issue#2

Document for:	Approval

Agenda item:	5.19

Spec:	3GPP TS 33.700-22

Version:	0.5.0

Work Item:	FS_CAPIF_Ph3-sec

Comments

This pCR proposes to update the conclusion for key issue#2.

SA6 has specified both authorization and access control policy. In clause 8.25.3.7 of TS 23.222, the procedure for API invoker obtaining authorization for service API access in CAPIF interconnection. The pre-condition specifies that the API invoker and the CCF-B are in the same trusted domain, the AEF and the CCF-A are in the same trusted domain and the CCF-A is the authorization function for service API access on the AEF. After successful authentication validation of the API invoker, if the CCF-B determines that the service API access authorization cannot be done by itself alone, then the CCF-B sends an interconnection obtain API authorization from the CCF-A.

In clause 8.25.3.9 of TS 23.222, the procedure for obtaining access control policy in CAPIF interconnection. This access control policy does not relate to the authorization information. Table E-1 in TS 23.222 specifies it as the volume limit (total number of invocations allowed), time limit (time range of the day during which the service API invocations are allowed) and rate limit (allowed service API invocations per second) which one CCF can learn from other CCF.

As in conclusion of key issue#3, the term authorization information is used which cannot be learnt from one CCF to another CCF. Hence it is proposed to remove the following Editor’s Note.

Editor’s Note: Whether CCF-B can learn authorization information from CCF-A is FFS.

* * * First Change * * * *

### 7.2.3	Conclusion for API invoker authentication and authorization mechanism

For mutual authentication and authorization between API invoker (on-boarded to CCF-A) and the AEF (registered to CCF-B), the procedures as defined in clause 6.5.2 of TS 33.122 [4] can be re-used with the following enhancement:

-	When using TLS-PSK or PKI:

-	On receiving the request from the AEF, CCF-B requests the security information (AEFPSK/root CA) from CCF-A (over CAPIF-6/6e reference point).

-	The AEF learns the authorization information from CCF-B.

-	When using TLS with OAuth token:

-	On receiving the request from the AEF, CCF-B requests the security information (e.g., root CA) from CCF-A (over CAPIF-6/6e reference point).

-	CCF-A can send the access token request to CCF-B.

-	CCF-B can provide an access token to the API invoker via CCF-A as specified in clause 6.5.2.3 in TS 33.122 [4].

-	The AEF verifies the access token as described in 6.5.2.3 in TS 33.122 [4].

-	Whether CCF-A can issue an access token and whether CCF-B needs to verify CCF-A is authorized for the authorization decision are to be determined during normative work.

-	For the case of TLS-PSK or PKI, and TLS with OAuth token methods listed above, the specific details of how the CCF-B requests the security information of the API Invoker from the right CCF-A, i.e. where API invoker is onboarded, are up to normative work.

-	For the case of TLS with OAuth token methods, how CCF-A determines to request token from CCF-B is up to normative work.

-	Further details of the procedure are to be determined during normative work.

* * * End of Change * * * *