3GPP TSG-SA3 Meeting #120	S3-250820

# Athens, Greece, Feb 17 – 21, 2025

Source:	OPPO

Title:	Update Solution #17 in TR 33.713

Document for:	Approval

Agenda item:	5.9

Spec:	3GPP TR 33.713

Version:	0.6.0

Work Item:	FS_Ambient_IoT_Sec

Comments

This pCR proposes to update the Solution #17.

Proposed Changes

* * * Start of Change * * * *

6.17	Solution #17: Disabling operation procedure for AIoT services

6.17.1	Introduction

The solution addresses the security requirement of KI#1: Protection for disabling device operation. According to TS 22.369 [2], the network operator shall provide a suitable mechanism to temporarily/permanently disable the capability of an Ambient IoT device or a group of Ambient IoT devices to transmit RF signals. The solution assumes the network operator is responsible for managing the AIoT device using the temporarily/permanently disable operation or the enable operation.

The disabling operation is implemented in the Inventory and Command procedure. The AIoTF calculates the integrity parameter (i.e., MAC) and optionally encrypt the disable command based on the pre-shared key or the derived session key between AIoT devices and AIoTF. The AIoT device verifies the integrity parameter and optionally decrypt the disable command using the same key to decide whether to perform the disable operation.

For the enabling operation, the enable command is carried in the Inventory only procedure. The AIoTF calculate the integrity parameter (i.e., MAC) or optionally encrypt the enable command based on the pre-shared key. The AIoT device verifies the enable security parameter and optionally decrypt the enable command using the same key to decide whether to perform the enable operation.

6.17.2	Solution details

6.17.2.1	Disable an AIoT device temporarily or permanently

Depicted in Figure 6.17.2-1 is the disable operation procedure for AIoT Services.

Figure 6.17.2.1-1: Disable operation procedure for AIoT service

- The AF sends the AIoT Disable Request to the AIoTF via the NEF, including the AIoT device filter information, and disable type (i.e., temporarily/permanently disable).

- The AIoTF sends the AIoT Disable Trigger to the RAN/UE Reader, including the AIoT device filter information.

The AIoT device filter information is used to page an AIoT device or a group of AIoT device, which could be the AIoT device ID, the group ID, or the partial ID of AIoT device.

- The RAN/UE Reader sends the AIoT Paging Request to the AIoT device, including the AIoT device filter information.

- The AIoT device checks whether the ID matches according to the AIoT device filter information.

- The AIoT device sends the AIoT Paging Response to the RAN/UE Reader, including the AIoT device ID.

- The RAN/UE Reader sends the AIoT Disable Trigger Response to the AIoTF, including the AIoT device ID.

- The AIoTF obtain the key to protect the enable operation from the ADM/AAA-S when it does not have the key for the AIoT device.

NOTE1: The key K can be the pre-shared key or the session key derived from pre-shared key.

- The AIoTF uses f function to calculate the integrity parameter with the input parameters key K, AIoT ID, disable type and freshness value (e.g., counter), i.e., f (K, AIoT ID, disable type, freshness value). If counter is adopted for freshness value, the initial value of the counter is set to zero and AIoTF increases the counter by one for each subsequent disable operation. When the counter overflows, the AIoTF resets the counter to zero. Optionally, the disable type can also be encrypted by the AIoTF using the obtained key.

NOTE2: The f functions can be HMAC functions, f1-f5 functions, AES functions, etc.

- The AIoTF sends the AIoT Disable Request to the RAN/UE Reader. The message contains the AIoT ID, integrity parameter, optionally encrypted disable type and freshness value (counter).

- The RAN/UE Reader forwards the AIoT Disable Request to the AIoT device.

- If counter is adopted for freshness value, the AIoT device is required to store the counter. When the received counter in the AIoT Disable Request is less than the local counter, the device refuses the Request, otherwise the AIoT device uses the same key to verify the integrity parameters and optionally decrypt the disable type.

- (a) The AIoT device performs the disable operation according to the disable type, if the calculated integrity parameter matches the received integrity parameter.

(b) The AIoT device sends the AIoT Disable Response to the RAN/UE Reader before disabling the RF capability, if the AF requires a feedback.

- The RAN/UE Reader forwards the AIoT Disable Response to the AIoTF.

- The AIoTF sends the AIoT Disable Response to the AF via the NEF.

6.17.2.2	Enable a temporarily disabled AIoT device

The following figure shows the call flow for securely enabling a temporarily disabled AIoT device.

Figure 6.17.2.2-1: Enable operation procedure for AIoT service

- The AF sends the AIoT Inventory Request message to the AIoTF via the NEF, which includes the AIoT device filter information, and enable command.

NOTE3: Whether the Enable Command is carried in the Inventory procedure depends on the size of the paging message defined by RAN2.

- The AIoTF obtain the key to protect the enable operation from the ADM/AAA-S when it does not have the key for the AIoT device. The Key Request message may contain the AIoT device ID and the freshness parameter for key derivation.

NOTE3: The key K can be the pre-shared key.

- Same as described in clause 6.17.2.1 step 8, the AIoTF uses the obtained Key to protect the Enable Command.

- The AIoTF sends the AIoT Inventory Request to the AIoT device via RAN/UE Reader. The request message contains the AIoT ID, integrity parameter, optionally encrypted enable command and freshness value.

- (a) The temporarily disabled AIoT device derive the key for protecting enable command with the input parameter pre-shared key and freshness value.

(b) The AIoT device uses the obtained key to verify the integrity parameters and optionally decrypt the enable command.

(c) The AIoT device performs the enable operation if the calculated integrity parameter matches the received disable security parameter

- The AIoT device sends the AIoT Inventory Response to the AIoTF via RAN/UE Reader.

- The AIoT NF sends the AIoT v Response to the AF via the NEF

6.17.3	Evaluation

This solution fulfils the security requirements in key issue #1.

This solution addresses the security of the Disable Command Operation by protecting the Disable Command carried in the Inventory and Command procedure based on the pre-shared key or the session key derived from pre-shared key .

This solution addresses the security of the Enable Command Operation by protecting the Enable Command carried in the Inventory only procedure based on the pre-shared key.

* * * End of Change * * * *