3GPP TSG-SA3 Meeting #120	S3-250838

Athens, Greece, 17 - 21 February 2025

Title:	[Draft] Reply LS on security questions related to NAS layer overhead reduction for data transfer using control plane CIoT

Response to:	LS S3-250316/C1-247156 on security questions related to NAS layer overhead reduction for data transfer using control plane CIoT from CT1

Release:	Release 19

Work Item:	NORDAT-CP

Source:	Xiaomi [to be SA3]

To:	CT1

Cc:

Contact person:	Jouy Shang

shangzhengyi@xiaomi.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1	Overall description

SA3 would like to thank CT1 for their LS on NAS layer overhead reduction for data transfer using control plane CIoT. SA3 would like to provide the following feedback.

Question 1: Currently, SERVICE REQUEST message in EPS NAS uses short MAC and short sequence number. Would it be acceptable to use short MAC and/or short sequence number for the new NAS message format with minimized overhead for data transfer via NAS?

Answer 1: Short MAC will reduce the security of NAS signalling. In the EPS, SERVICE REQUEST message is used to trigger the establishment of a NAS signalling connection and does not contain any UE data. For transferring NAS signalling contained UE data, SA3 suggests using regular MAC (4 octets).

For the short sequence number (5 bits), if the receiver is able to estimate the remaining 3 most significant bits of the sequence number, it is acceptable to use short sequence number for the new NAS message format with minimized overhead.

Question 2: Currently, all initial NAS messages contain Key Sequence Indicator (KSI). Would it be acceptable not to include the KSI in the new initial NAS message format with minimized overhead for data transfer via NAS?

Answer 2: No. The KSI is used to identify the used NAS key set and is mandatory to be included.

Question 3: In the NAS message format with minimized overhead for data transfer via NAS, only a portion of the message will be ciphered. Is it acceptable for the ciphered portion not to be byte-aligned? Not having to byte-align the ciphered portion would allow for more efficient message encoding.

Answer 3: Using non-byte-aligned ciphering has impact on UE implementations which currently all use byte-aligned ciphering, particularly to accommodate hardware implementations requiring byte alignment. Therefore, byte-aligned ciphering is preferred to simplify UE implementation.

2	Actions

To CT1

ACTION: 	SA3 kindly asks CT1 to take the above information into account.

3	Dates of next TSG SA WG 3 meetings

SA3#121	7 – 11 April 2025		Goteborg, Sweden

SA3#122	19 - 23 May 2025		Fukuoka, Japan