3GPP TSG-SA3 Meeting #120	S3-250889

# Athens, Greece, 17 - 21 February 2025

Source:	Nokia, Nokia Shanghai Bell

Title:	Update conclusion for KI#3 Security aspects of digital asset container in 5G

Document for:	Approval

Agenda item:	5.18

Spec:	3GPP TR 33.721

Version:	0.6.0

Work Item:	FS_Metaverse_Sec

Comments

<Proposals, reason for change, abstract, comments if necessary (optional)>

This pCR proposes to update conclusion for KI#3 on Security aspects of digital asset container in 5G.

The solutions in the present TR propose extending the existing security mechanism for SEAL or CAPIF to authenticate and authorize a digital client for digital asset creation and access. The conclusion of the KI#3 in the TR supposed CAPIF security defined in TS 33.122 [5] shall be followed for authorizing a digital asset requestor (API Invoker) to access service API provided by the digital asset server. When DA server has sufficient local information to authorize the digital asset requestor to access digital assets stored in the DA server, whether CAPIF needs to be used for digital asset access control is FFS.

However, the DA server may not have sufficient local information for authorization of the DA requestor. In addition, in CAPIF, also SEAL security, OAuth2.0 access token is used to authorize the API invoker to access the service/resource provided by the AEF, not necessarily the user behind the API invoker. E.g. an owner of the resource may have full control of the resource, but it doesn’t mean the application client using by the resource owner can have full control on the resource.

On the other hand, in TR 33.700-22 v0.5.0, it concluded in KI#1.3 that Normative work is recommended to support service operation level and resource level granularity in RNAA scenarios. So when user authorization for the API Invoker is not available locally, RNAA shall be reused to get authorization from the owner of digital asset and operation & resource level granularity shall be supported by CAPIF for DA access. The first EN in the conclusion will be updated accordingly.

When access control list is included in the DA profile, the DA server may further check if the user behind the API invoker can access the resource/DA. However, user management and authentication is not supported by CAPIF so far. The access control list may not be applicable to CAPIF unless the user id in the allowed user list can be mapped to a VAL UE ID/GPSI. An EN will be added on this aspect.

If SEAL security defined in TS 33.434 is reused to authorize a digital asset requestor (e.g. DA client) to access service API provided by the digital asset server, first the solution should refer to the SEAL architecture supporting DA defined in TS 23.438 and be able to map the entities in the DA access authorization workflow to roles in SEAL security. Second as the SEAL security is based on OIDC to authenticate the VAL user and grant a VAL/SEAL application to access service/resource produced by a VAL/SEAL server based on the authorization of the user, the solution should be able to map the entities in the DA access authorization workflow to OIDC roles. Third the solution should clarify who/which entity should be authenticated and who/which entity should be authorized to access the DA. Fourth when access control list is included in the DA profile, the DA server should further check if the user behind the DA requestor/client can access the resource/DA based on allowed users of the access control list.  Based on those principles, authorization of DA requestor to access the digital asset(s) based on SEAL security defined in 33.434 is FFS.

Proposed Changes

* * * First Change * * * *

<Proposed change in revision marks>

# 7	Conclusions

## 7.3	Conclusion on Key Issue #3

The TS 23.438 [8] specifies in sufficient detail the requirements, architecture, procedures and APIs for digital asset management, the digital asset service is part of the SEAL services specified in 3GPP TS 23.434 [7].

The normative work of the KI#3 aligns with the architecture of digital asset management defined in the TS 23.438 [8] and the below security solution principles should be followed:

- Security for the SEAL interfaces, especially SEAL-S, SEAL-UU, specified in TS 33.434 [4] is applied for protection of digital asset management interfaces such as DA-S, DA-UU as specified in TS 23.438 [8].

- When CAPIF is used as specified in TS 23.434 [7], the security mechanism for CAPIF specified in TS 33.122 [5] shall be followed for authenticating and authorizing a digital asset requestor (API Invoker) to access the digital asset service.

Editor’s note: When CAPIF is used as specified in TS 23.434 [7], authorization of DA requestor (API Invoker) at operation and resource level shall be aligned with TR 33.700-22 and normative work of CAPIF phase 3 security.  .

Editor’s note: When CAPIF is used as specified in TS 23.434 [7], how to check if the user behind the DA requestor (API invoker) is allowed to access the digital asset based on access control list included in the DA profile is FFS.

Editor’s note: Authorization of DA requestor to access the digital asset(s) based on SEAL security defined in 33.434 is FFS

Editor’s note: Further conclusions are FFS.

* * * End of Changes * * * *