3GPP TSG-SA3 Meeting #120	S3-250940

Athens, GR, 17 -21 February 2025

Source:	Google, AT&T, TELUS

Title:	pCR to ACME-SBA living document on: certificate enrollment and renewal in 5G SBA using ACME

Document for:	Approval

Agenda item:	4.18

Spec:	3GPP TS 33.310

Version:	19.3.0

Work Item:	ACME_SBA

Comments

This document proposes to add the procedures for certificate enrollment and renewal in 5GC SBA using ACME.

Proposed Changes

* * * First Change * * * *

## YY.4	Certificate enrolment and renewal for 5GC NFs

The ACME certificate enrolment (issuance) and renewal for the 5GC SBA are based on RFC 8555 [XX] with the following modifications:

- Pre-authorization in RFC 8555 are not used. In addition:

  - The newAuthz resource should not be supported.

  - Authorization objects should be created in the pending state, and only transition to valid after successful completion of the challenge validation as described in clause YY.5 (Challenge validation)

ACME supports X.509 version 3 certificates and the necessary extensions. Table YY.4.1 lists the ACME challenge type tkauth-01 [XB] as defined in Annex YY clause YY.5 used for each 5G SBA certificate profile.

Table YY.4.1: ACME challenge types per 5G SBA certificate profile

| 5G SBA certificate type | ACME challenge types |
| --- | --- |
| TLS client | tkauth-01 |
| TLS server | tkauth-01 |
| OAuth 2.0 access token | tkauth-01 |
| CCA token | tkauth-01 |

Figure YY.3.1 describes the message flow for certificate enrolment and certificate renewal procedure.

Figure YY.3.1 ACME Certificate enrolment and renewal procedure in 5GC SBA

- The 5GC NF, which acts as an ACME client,  requests a certificate by sending a new order request for 5G SBA ACME Identifier to the CA’s (ACME server’s) newOrder resource using a POST request. A 5GC SBA ACME Identifier is described in clause YY.1.X.

A set of pre-configured policies in the 5GC NF automatically triggers the 5GC NF to send a certificate enrolment or a certificate renewal request. The certificate renewal policies may be configured based on recommendation in Annex I.2

- The CA replies to the 5GC NF with a 201 (Created) response that includes authorization objects for validation challenge. The valid challenge type(s) suitable for 5G SBA are limited to the ones specified in YY.5.

- The 5GC NF checks the authorization objects within the response and completes the listed challenges before requesting the CA to sign the certificate. If the challenge is not completed successfully before the expiration time initially provided by the CA or updated by the CA, the 5GC NF may start a wait timer and restart the certificate renewal procedure from Step 1. The wait timer may be pre-configured with an appropriate wait time (e.g., 0 seconds).

Additionally, it should be appreciated that authorization object expiry times are configured according to operator policy. If an authorization object is in a valid state and has not expired, the 5G NF doesn't need to perform challenge validation for the corresponding identifier in subsequent orders.

- After the 5GC NF successfully completes the challenge validation procedure, it sends a Certificate Signing Request (CSR) to the CA.

Note that after several attempts, if the challenge is not completed successfully prior to the expiration time initially provided by the ACME server  (or updated by the ACME server), the order will eventually be dropped by the ACME server.

- The CA issues the certificate and publishes it in the corresponding resource directory to the URL provided in    the order object.

- The 5GC NF downloads the certificate by sending a POST-as-GET request to the certificate URL provided.

* * * Second Change * * * *

2	References

[XB]                       “[XX]  IETF RFC 9447: "Automated Certificate Management Environment (ACME) Challenges Using an Authority Token".”

* * * End of Changes * * * *