3GPP TSG-SA3 Meeting #120	S3-251042

Athens, Greece, 17 - 21 February 2025

# Source:	Qualcomm Incorporated, Huawei, HiSilicon, China Telecom Corporation Ltd., Xiaomi Technology Spain S.L

# Title:	U2N relay discovery security

# Document for:	Approval

Agenda Item:	4.15

# 1	Decision/action requested

This contribution propose text for U2N relay discovery security.

# 2	References

[1]	TR 33.743

[2]	Skeleton for the draft CR

# 3	Rationale

This contribution proposes text for U2N relay discovery security based on the conclusions in the TR 33.743 [1].

Merge of 250435, 250679, 250632, and 250841.

# 4	Detailed proposal

It is proposed that the below changes to the draft CR [2] are approved.

# * * * * First change * * * *

##### 6.1.3.X.3	Security flows for 5G ProSe Multi-hop UE-to-Network Relay Discovery

###### 6.1.3.X.3.1	Discovery with Model A

The security procedure for multi-hop UE-to-Network Relay discovery with Model A is shown in Figure 6. 1.3.X.3.1-1.

Figure 6.1.3.X.3. 1-1: Model A Discovery operation supporting multi-hop UE-to-Network Relay

0.	The 5G ProSe Remote UE, Intermediate UE-to-Network Relay/5G ProSe UE-to-Network Relay is provisioned with the discovery security materials associated with an RSC from the 5G PKMF/5G DDNMF of 5G ProSe Remote UE/Intermediate UE-to-Network Relay/5G ProSe UE-to-Network Relay’s HPLMN based on the procedure specified in clause 6.1.3.2. The discovery security materials shall contain a Discovery User Integrity Key (DUIK) for the integrity protection of Relay Discovery Announcement.

Editor’s Note: Discovery security materials provisioning for inter-PLMN scenario is FFS.

1.	The 5G ProSe UE-to-Network Relay shall protect a Relay Discovery Announcement using the discovery security materials associated with the RSC as specified in clause 6.1.3.2. Then, the 5G ProSe UE-to-Network Relay broadcasts the Relay Discovery Announcement.

2a.	The Intermediate UE-to-Network Relay shall process the received Relay Discovery Announcement message using the discovery security materials associated with the RSC as specified in clause 6.1.3.2. If the processing is successful and Intermediate UE-to-Network Relay does not have a PC5 link with the 5G ProSe UE-to-Network Relay, the Intermediate UE-to-Network Relay shall establish a PC5 link with the 5G ProSe UE-to-Network Relay based on the PC5 security establishment for 5G ProSe UE-to-Network relay communication specified in clause 6.3.3.

2b.	Once the PC5 link is established between the Intermediate UE-to-Network Relay and the 5G ProSe UE-to-Network Relay, the Intermediate UE-to-Network Relay shall update the path information (e.g., hop count, Relay Info.) and protect the updated message using the discovery security materials associated with the RSC as specified in clause 6.1.3.2. The Intermediate UE-to-Network Relay broadcasts the updated message.

3.	Upon receiving the Relay Discovery Announcement message from the Intermediate UE-to-Network Relay, the 5G ProSe Remote UE shall process the received message using the discovery security materials associated with the RSC as specified in clause 6.1.3.2.

###### 6.1.3.X.3.2	Discovery with Model B

The security procedure for multi-hop UE-to-Network Relay discovery with Model B is shown in Figure 6.1.3.X.3. 2-1.

Figure 6.1.3.X.3. 1-1: Model B Discovery operation supporting multi-hop UE-to-Network Relay

0.	The 5G ProSe Remote UE, Intermediate UE-to-Network Relay/5G ProSe UE-to-Network Relay is provisioned with the discovery security materials associated with an RSC from the 5G PKMF/5G DDNMF of 5G ProSe Remote UE/Intermediate UE-to-Network Relay/5G ProSe UE-to-Network Relay’s HPLMN based on the procedure specified in clause 6.1.3.2. The discovery security materials shall contain a Discovery User Integrity Key (DUIK) for the integrity protection of Relay Discovery Solicitation and Relay Discovery Response.

Editor’s Note: Discovery security materials provisioning for inter-PLMN scenario is FFS.

1.	The 5G ProSe Remote UE shall protect a Relay Discovery Solicitation using the discovery security materials associated with the RSC as specified in clause 6.1.3.2. Then, the 5G ProSe Remote UE broadcasts the Relay Discovery Solicitation.

2.	The Intermediate UE-to-Network Relay shall process the received Relay Discovery Solicitation using the discovery security materials associated with the RSC as specified in clause 6.1.3.2. If the processing is successful, the Intermediate UE-to-Network Relay shall update the path information (e.g., hop count) and protect the updated message using the discovery security materials associated with the RSC as specified in clause 6.3. Then, the Intermediate UE-to-Network Relay broadcasts the message.

3.	Upon receiving the Relay Discovery Solicitation from the Intermediate UE-to-Network Relay, the 5G ProSe UE-to-Network Relay shall process the received message using the discovery security materials associated with the RSC as specified in clause 6.1.3.2. If the processing is successful, the 5G ProSe UE-to-Network Relay shall construct a Relay Discovery Response and protect it using the discovery security materials associated with the RSC as specified in clause 6.1.3.2.

The 5G ProSe UE-to-Network Relay replies to the Intermediate UE-to-Network Relay with the Relay Discovery Response.

4.	Upon receiving the Relay Discovery Response from the 5G ProSe UE-to-Network Relay, the Intermediate UE-to-Network Relay shall process the received message using the discovery security materials associated with the RSC as specified in clause 6.1.3.2. If the processing is successful, the Intermediate UE-to-Network Relay shall update the path information (e.g., hop count) and protect the updated message using the discovery security materials associated with the RSC as specified in clause 6.1.3.2. Then, the Intermediate UE-to-Network Relay replies to the 5G ProSe Remote UE with the message.

5.	Upon receiving the Relay Discovery Response from the Intermediate UE-to-Network Relay, the 5G ProSe Remote UE shall process the received message using the discovery security materials associated with the RSC as specified in clause 6.1.3.2.

# * * * * End of change * * * *