3GPP TSG-SA3 Meeting #120	S3-251061

# Athens, Greece, 17 - 21 February 2025

Source:	Huawei, HiSilicon, Nokia

Title:	Authorization for selection of VFL participants in VFL Group (External AF act as VFL Server)

Document for:	Approval

Agenda item:	4.24

Spec:	3GPP TS 33.501

Version:	19.1.0

Work Item:	AIML_CN_SEC

Comments

The proposal aims to add authorization handling for VFL members while participating in the same VFL process in clause X.12. The authorization for selecting VFL members i.e NWDAF/AF in the same VFL process utilises NRF token-based authorization as specified in clause 13.4.1 for the participating in VFL procedure.

This change is covered in X.12.2.2 when external AF act as VFL server. Hence it is proposed to add the authorization aspect for VFL members in this change.

* * * First Change * * * *

### X.12.2.2	Authorization of candidate VFL participants for vertical federated learning when external AF is acting as the VFL server

Figure X.12.2.2-1 depicts the authorization mechanism for NWDAF as VFL Client and external AF as VFL server. The authorization is based upon the VFL capability information per supported Analytics ID which includes VFL capability type (VFL server and/or VFL client), Analytics ID and VFL Interoperability Indicator per Analytics ID provided by VFL members during registration.

Figure X.12.2.2-1: Authorization of candidate VFL participants for VFL when external AF is acting as the VFL server

1. In case of AF as VFL Server, the NEF update its NF profile as specified in clause 6.2H.2.1.2 of TS 23.288[105], which including AF’s VFL capability information per supported analytics ID which includes VFL Capability type i.e. (VFL Client) and VFL interoperability indicator(s) per analytics ID and associated AF ID. In case of NWDAF as VFL Client it shall register to NRF with its NF profile as specified in clause 6.2H.2.1.1 of TS 23.288[105].

2. The AF sends a Discovery_Request to the NEF with information Analytics ID, VFL capability type (VFL Client), VFL interoperability indicator to discover for VFL Client. The NEF discovers NWDAF as VFL client from the NRF by invoking the Discovery_Request as specified in 6.2H.2.1.2 of TS 23.288[105]. The NEF checks based on configured policies whether the AF is entitled to discover service to a VFL client as specified in clause 12 of TS 33.501.

NOTE: Handling of temporary NWDAF instance ID (s) is specified in clause 6.2H.2.1.2 of TS 23.288 [105].

3. The AF acting as VFL server sends VFLService_Request towards NEF. The Service request shall contain the Analytics ID and optionally VFL interoperability indicator per analytics ID.

4. The NEF request for an access token to the NRF as specified in clause 13.4.1 to perform VFL service towards the VFL clients (NWDAF). The token request shall contain the Analytics ID, AF ID and optionally VFL interoperability indicator per analytics ID.

5. The NRF checks whether NEF is authorized to access the NWDAF for the VFL service(s) by optionally verifying the VFL interoperability indicator in the token request is in the VFL interoperability indicator(s) as received from VFL server and VFL client in in Step 1.

6. If the NEF is authorized, the NRF will issue an access token in response. The token claim may also include VFL interoperability indicator for that particular analytics ID, Analytics ID and AF ID.

7. The NEF send VFLService_Request to NWDAF (VFL Client) and the token includes the Analytics ID, AF ID and optionally VFL interoperability indicator per analytics ID as token received in Step 6.

8. The NWDAF verifies the received access token as specified in clause 13.4.1 of TS 33.501 and checks for the token claims as received in step 7.

9a. In case of successful access token verification, the NWDAF (VFL as client) sends a VFLService_Response to the NEF.

9b. The NEF sends the VFLService_Response to the AF (VFL as Server). Protection of NEF-AF interface is as specified in clause 12 of TS 33.501.

* * * End of Changes * * * *