3GPP TSG-SA3 Meeting #120	S3-251083

# Athens, Greece,  17 - 21 February 2025

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.558 | CR | draftCR | rev | 3 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Solutions for secure retrieval of 5G system UE Ids and privacy related information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGE_Ph3 |  |  |  |  |  | Date: |  |  | 2024-02-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The KI#2.1 of TR 33.749 (Secure retrieval of 5G system UE Ids and privacy related information) was concluded by a collection of solution design principles to be followed in normative text, and by indicating that alternative solutions with those design principles would be captured in an informative annex during normative work. This CR compiles the AKMA based mechanism. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New annex which compiles alternative solutions for secure retrieval of 5G syterm UE Ids and privacy related information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There would not be illustrative examples of implemenation of the security principles agreed for security retrieval of 5G system UE ids and privacy related information. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New annex X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S3-250471 |  |  |  |  |  |  |  |  |

* * * Start of changes * * * *

## X.Y	AKMA based mechanism

### X.Y.1	Overview

The main design principle of the solution is based on AKMA feature, more specifically on the A-KID, an application specific ID derived from the long term symmetric pre-shared key K between the UE and the network. The EDGE application server (EAS) and/or enabler server (EES), use the AKMA protocol to retrieve the 5G UE privacy related information.

The solution assumes that AF (EES/EAS) communicates with 5GC through NEF.

### X.Y.2	Details

Figure X.2-1 illustrates the procedure to fetch the 5G system UE privay related information based on AKMA.

Figure X.Y.2-1: Procedure to fetch the 5G system UE privacy information based on AKMA

Preliminary steps:

- Primary authentication
- AKMA Key derivation (A-KID) as per procedure in TS 33.535 [11].
- Creation of AKMA context in AAnF (SUPI, KAKMA, A-KID) as per procedure in [11].

Step 1. The application client in the UE (EEC) requests a service to the AF, i.e., EDGE application server (EAS, EES), including the A-KID (AKMA Key Identifier) in the request.

Step 2-5. AKMA procedure intended to provide KAF to the AF from the AAnF via NEF as specified in clause 6.3 of [11].

NOTE: AF specific UE Id does not need to be part of the of the incoming request (step 2), since the NEF will provide the GPSI (external ID) to AF.

Step 6. The AF requires to know privacy information of the UE available in the 5G Core, such as the the location of UE, and makes the request to the 5G Core via NEF, using the given GPSI as UE identifier.

Step 7. The NEF checks the GPSI and if the request is authorized, the corresponding information, in this example location of the UE, is provided to the AF.

Step 8. The AF responds to the UE (EEC).

NOTE:	Special consideration can be needed for the point that the same A-KID will be used until the next execution of the primary authentication, and it will be same for all the AFs.

* * * End of Changes * * * *