3GPP TSG-SA3 Meeting #120	S3-251107

# Athens, Greece, 17 - 21 February 2025

Source:	Philips International B.V.

Title:	Pseudo-CR on Solution#1 update - Addressing ENs

Document for:	Approval

Agenda item:	5.9

Spec:	3GPP TR 33.713

Version:	0.6.0

Work Item:	FS_AIoT_Sec

Comments

Regarding Editor’s Note: Whether the solution aligns with SA2 system architecture and procedures is FFS.

As SA2 could not reach a conclusion regarding the temporary disabling and enabling features, these will not be supported in Release 19. As such, the solution herein would not align with SA2 procedures to be specified in Release 19. This is captured in the evaluation section. 

As no further evaluation is foreseen, the Editor’s Note: Further evaluation is FFS is removed.

* * * First Change * * * *

## 6.1	Solution #1: Ambient IoT device disabling mechanism

### 6.1.1 	Introduction

This solution addresses KI#1.

According to TS 22.369, the enable/disable device operations are used by the network operator to manage the Ambient IoT device’s capability to transmit RF signals. As the disabling of RF transmission capability could, according to the operator’s policy, be temporary or permanent, it is paramount to ensure that the disabling, specifically of a permanent nature, is performed securely and in a manner that allows device recovery in case the system was compromised, and an attacker has managed to issue “disable” commands to one or multiple Ambient IoT devices.

### 6.1.2 	Solution details

Figure 6.1.1 – Ambient IoT device disabling mechanism

The permanent disabling of an AIoT device is performed in a two-stage operation, where initially, the AIoT device is temporarily disabled, and then, following a cool-down period (i.e., recovery time window), the AIoT device could be disabled permanently. The two-stage permanent disabling operation is performed as follows:

In Step 0, the AIoT device is provisioned with a configuration determining how the device processes disabling operations. The configuration includes the required cool down period that needs to be met before a permanent “disable” command is allowed.

In Step 1a, the authentication procedure is performed between the AIoT device and the AIoT Management Function. Upon a successful run of the authentication procedure,, the AIoT managing function issues a temporary disable command to the Ambient IoT device in step 1b. The command includes a counter T1.

NOTE 1: The AIoT Management Function (AIoT MF) depends on the entity that owns or manages the resource (i.e., AIoT device). If a device is owned or managed by the network, the disabling is triggered by the CN function AIoTF, otherwise, it is triggered by a 3rd party Management Function (e.g., AF) managing the device.

In Step 2, The AIoT device, upon receiving the temporary disable command, retrieves and stores the counter T1, which will be used in subsequent processing.

In Step 3, the AIoT device sends an ACK to the AIoT MF, which may contain the counter received in Step 0. Then, the AIoT device temporarily disables its RF transmission capability.

NOTE 2: Following step 3, the AIoT device is limited to only performing a re-enable, or a permanent disable action.

In Step 4a, The AIoT MF pages the AIoT device and performs a mutual authentication procedure. Following a successful authentication and depending on whether the AIoT MF intends to re-enable the AIoT device (i.e., recovery scenario), or permanently disable the AIoT device. The AIoT MF sends in:

-  Step 4b: a message containing an enable command to recover the AIoT device, or

- Step 4c: a message containing a permanent disable command, in which a second counter T2 is included.

NOTE 3: The messages in step1 and step3 carrying the temporary and permanent disable commands are protected using the same means of protection applicable to other commands (e.g., write), and so is the verification that the disable commands are coming from a legitimate party.

In Step 5, The AIoT device processes the received command. If the AIoT is requested to enable its RF transmission capability, the AIoT device recovers from the temporary disabled state and discards the stored counter T1. Otherwise, if the AIoT device is requested to perform a  permanent disable command, it retrieves the second counter from the message, then checks whether the following conditions are met:

- The RF transmission capability is temporarily disabled.

- Whether the value T2 – T1 is greater or equal to the cool-down period configured in the AIoT device.

If the checks succeed, the AIoT device temporarily enables its RF transmission capability to send an ACK message to the AIoT MF in step 6 confirming that the device is permanently disabling its RF transmission capability.

In Step 7, the AIoT device disables its RF transmission capability permanently.

* * * Next Change * * * *

### 6.1.3 	Evaluation

To mitigate the threat of unauthorized permanent disabling (e.g., through spoofed commands), the solution involves a controlled, two-stage operation for permanent disabling. By enforcing the cool-down period, the system adds an extra layer of control, allowing time for validation and potential recovery if an unauthorized command is detected.

In the solution proposed, the protection of disable commands (temporary and permanent) re-uses the same means of protection applicable to other commands (e.g., write command).

Ambient IoT device(s) need to maintain an internal state to support the security mechanism proposed by the solution.

Prior to re-enabling or permanently disabling a temporarily disabled Ambient IoT device, a mutual authentication procedure is performed. While temporarily disabled, the Ambient IoT device is limited to only performing a re-enable/permanent disable action.

The solution proposed does not align with the conclusions of TR 23.700-13, which state that the temporary disabling and enabling feature will not be supported in Release 19.

* * * End of Changes * * * *