3GPP TSG-SA3 Meeting #120	S3-251137

Athens, Greece, 17 - 21 February 2025

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | 2093 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Requirements for secure retrieval of 5G system UE Ids and privacy related information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGE_Ph3 |  |  |  |  |  | Date: |  |  | 2024-02-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The KI#2.1 of TR 33.749 related to the security retrieval of 5G system UE Ids and privacy related information was concluded with the agreement of a set of solution design principles to be followed in normative phase. This CR implements those principles as requirements specifically for the Nnef_UEId API. Please note that in the procedures described in clause 6 of TS 33.558 it is stated that the security aspects related to NEF APIs when they are selected for interfaces EDGE-2/7/8 are to be covered in clause 12 of TS 33.501 (Security aspects of NEF). Therefore to be consistent with the specificationsit is proposed this CR for TS 33.501, as it refers to a NEF API. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New security requirements in the usage of Nnef_UEId API |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The usage of the Nnef_UEId API could compromise the security and privacy of the UE |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | It was S3-250481 |  |  |  |  |  |  |  |  |

* * * First Change * * * *

# 12	Security aspects of Network Exposure Function (NEF)

## 12.1	General

In the 5G system, the Network Functions securely expose capabilities and events to 3rd party Application Functions (AF) via NEF. The NEF also enable secure provision of information in the 3GPP network by authenticated and authorized AFs.

NOTE 1:	If a token is generated for AF authorization, such a token can include specific information depending on the procedure, e.g. clause 16.6.3.

NOTE 2:	It is recommended that the information used in the call invocation to NEF API "Nnef_UEId API" includes a non-permanent information as a reference to UE. The entity responsible for the generation of that information needs to be part of the 5G system in HPLMN, or alternatively that information can also be generated by UE. The 5G System in HPLMN should verify the information used in Nnef_UEId API call invocation.

Requirements on security aspects of NEF are captured in clause 5.9.2.3.

## 12.2	Mutual authentication

For authentication between NEF and an AF that resides outside the 3GPP operator domain, mutual authentication based on client and server certificates shall be performed between the NEF and AF using TLS.

Certificate based authentication shall follow the profiles given in 3GPP TS 33.310 [5], clause 6.1.3a. The identities in the end entity certificates shall be used for authentication and policy checks. The structure of the PKI used for the certificate is out of scope of the present document.

## 12.3	Protection of the NEF – AF interface

TLS shall be used to provide integrity protection, replay protection and confidentiality protection for the interface between the NEF and the AF. The support of TLS is mandatory.

Security profiles for TLS implementation and usage shall follow the provisions given in clause 6.2 of TS 33.210 [3].

## 12.4	Authorization of Application Function’s requests

After the authentication, NEF determines whether theAF is authorized to send requests for the 3GPP Network Entity. The NEF shall authorize the requests from AF using OAuth-based authorization mechanism, the specific authorization mechanisms shall follow the provisions given in RFC 6749 [43].

## 12.5	Support for CAPIF

When the NEF supports CAPIF for external exposure as specified in clause 6.2.5.1 in TS 23.501[2], then CAPIF core function shall choose the appropriate CAPIF-2e security method as defined in the sub-clause 6.5.2 in TS 33.122[53] for mutual authentication and protection of the NEF – AF interface.

* * * End of Changes * * * *