3GPP TSG-SA3 Meeting #120	S3-250619

# Athens, Greece 17 – 21 Feburary 2025

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | draft | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | General description of X.12 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN_SEC |  |  |  |  |  | Date: |  |  | 2025-2-17 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Add general description of X.12 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add general description of X.12 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No general description |  |  |  |  |  |  |  |  |
| Clauses affected: |  | X.12 (new clause) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Clause Annex α of this draft CR is only used to document the work on the different options considered for the security aspects of inter-CU LTM. This annex α will not be included when this draft CR is converted into CR. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Change

# X.12	Security for Vertical Federated Learning among NWDAFs and AFs

X.12.1	General

In case that external AF is VFL Server, and NWDAF(s) are VFL client(s) (NEF is involved), the following principles for authorization of participant NWDAF(s) for vertical federated learning shall be followed:

1)	The NEF includes requested analytics ID and AF ID in token request to NRF. The NRF checks whether the server AF is authorized to access the client NWDAF(s) for this Analytics ID. If the server AF is authorized, the NRF will issue an access token to the NEF. The NEF uses the access token to communicate with client NWDAF(s), and the NWDAF(s) verifies the access token, after successful verification, the NWDAF allows server AF’s request from the NEF.

2)	The NRF uses the information included in token request and registered information for the authorization.

In case that NWDAF is VFL Server, and external AF(s) are VFL client(s) (NEF is involved), the following principles for authorization of participant AF(s) for vertical federated learning shall be followed:

1)	The server NWDAF includes requested analytics ID and optionally client AF ID(s) in token request to NRF. The NRF checks whether the server NWDAF is authorized to access the NEF for this Analytics ID. If the server NWDAF is authorized, the NRF will issue an access token to the server NWDAF. The server NWDAF uses the access token to communicate with NEF, and the NEF verifies the access token, after successful verification, the NEF allows NWDAF’s request to external client AF(s).

2)	The NRF uses the information included in token request and NF profile for the authorization.

3)	Authorization of the AF is performed at the NEF, following clause 12 of the present document.

4)	The AF may use local policy to authorize VFL service request from NEF and not specified in 3GPP.

- 5)  Additionally, the access token may include the AF ID(s) and Analytics ID, and NEF may check whether the server NWDAF is allowed to access the requested AF based on the access token.

X.12.2	Authorization of candidate VFL participants

X.12.2.1	Authorization of candidate VFL participants for vertical federated learning when NWDAF is acting as the VFL server

Editor’s Note:	This clause provides description of Authorization of candidate VFL participants for vertical federated learning when NWDAF is acting as the VFL server, the SA2 procedure is defined in clause 6.2H.2.1.1 in TS 23.288, and the related conclusion is made in clause 7.2 in TR 33.784.

X.12.2.2	Authorization of candidate VFL participants for vertical federated learning when external AF is acting as the VFL server

Editor’s Note:	This clause provides description of Authorization of candidate VFL participants for vertical federated learning when external AF is acting as the VFL server, the SA2 procedure is defined in clause 6.2H.2.1.2 in TS 23.288, and the related conclusion is made in clause 7.2 in TR 33.784.

X.12.3	NEF security requirements

Editor’s Note:	This clause provides description of NEF security requirements, and the related conclusion is made in clause 7.2 and clause 7.3 in TR 33.784.

X.12.4	Protection of communication data used in VFL process.

Editor’s Note:	This clause provides description of protection of communication data used in VFL process, the SA2 procedure is defined in clause 6.2H.2.1, 6.2H.2.2, 6.2H.2.3, and 6.2H.2.4 in TS 23.288, and the related conclusion is made in clause 7.4 in TR 33.784.

End of Change