3GPP TSG-SA3 Meeting #120	S3-250655

Athens, Greece, 17 - 21 February 2025

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.784 | CR | 0002 | rev |  | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update conclusion on key issue#2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_AIML_CN_SEC |  |  |  |  |  | Date: |  |  | 2025-02-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As specified in clause 5.2 of TS 23.288, the NWDAF as VFL client or VFL server includes its VFL interoperability indicator per Analytics ID during registration to NRF. When NFs (i.e. NWDAF and/or AF) perform VFL together, only NFs sharing same VFL interoperability indicator can understand feature ID(s) and other VFL configurations from each other. It is proposed to add VFL interoperability indicator in the principles for authorization of participant NWDAF(s). The NRF shall authorizes the consumer based on the VFL interoperability indicator. Only the participant NWDAF(s) share same VFL interoperability indicator could perform VFL together. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add VFL interoperability indicator. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment with SA2 |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

7.2	Conclusion on Key issue #2

In case that external AF is VFL Server, and NWDAF(s) are VFL client(s) (NEF is involved), the following principles for authorization of participant NWDAF(s) for vertical federated learning are recommended for normative work:

1)	The NEF includes requested analytics ID, VFL interoperability indicator and AF ID in token request to NRF. The NRF checks whether the server AF is authorized to access the client NWDAF(s) for this Analytics ID and support same VFL interoperability indicator. If the server AF is authorized, the NRF will issue an access token to the NEF. The NEF uses the access token to communicate with client NWDAF(s), and the NWDAF(s) verifies the access token, after successful verification, the NWDAF allows server AF’s request from the NEF.

2)	The NRF uses the information included in token request and registered information for the authorization.

In case that NWDAF is VFL Server, and external AF(s) are VFL client(s) (NEF is involved), the following principles for authorization of participant AF(s) for vertical federated learning are recommended for normative work:

1)	The server NWDAF includes requested analytics ID, VFL interoperability indicator and optionally client AF ID(s) in token request to NRF. The NRF checks whether the server NWDAF is authorized to access the NEF for this Analytics ID and VFL interoperability indicator. If the server NWDAF is authorized, the NRF will issue an access token to the server NWDAF. The server NWDAF uses the access token to communicate with NEF, and the NEF verifies the access token, after successful verification, the NEF allows NWDAF’s request to external client AF(s).

2)	The NRF uses the information included in token request and NF profile for the authorization.

3)	Authorization of the AF is performed at the NEF, following clause 12 of TS 33.501 [5].

4)	The AF may use local policy to authorize VFL service request from NEF and not specified in 3GPP.

Additionally, the access token may include the AF ID(s), VFL interoperability indicator and Analytics ID, and NEF may check whether the server NWDAF is allowed to access the requested AF based on the access token..

In case that NEF is involved, the following principles for masking internal network information for VFL are recommended for NEF security requirements:

-	NF instance ID should not be transmitted outside the 3GPP operator domain via NEF.