3GPP TSG-SA3 Meeting #120	S3-250678

# Athens, Greece, 17 - 21 February 2025

Source:	Huawei, HiSilicon

Title:	Adding general introduction and security requirements of multi-hop UE-to-Network Relay to ProSe draftCR

Document for:	Approval

Agenda item:	4.15

Spec:	3GPP TS 33.503

Version:	18.5.0

Work Item:	5G_ProSe_Sec_Ph3

Comments

Accept the following modification to include general introductions and security requirements of multi-hop U2NW Relay to the ProSe Ph3 draft CR.

* * * First Change * * * *

6.1.1	General

This clause describes the security requirements and procedures that are specifically applied to 5G ProSe Discovery defined in TS 23.304[2].

The security requirements for 5G ProSe Discovery are defined in clause 6.1.2.

The security procedures for open 5G ProSe Direct Discovery are defined in clause 6.1.3.1, the security procedures for restricted 5G ProSe Direct Discovery are defined in clause 6.1.3.2, the security procedures for 5G ProSe UE-to-UE Relay Discovery are defined in clause 6.1.3.3. The security requirements and security procedures for 5G ProSe multi-hop UE-to-Network Relay Discovery are defined in clause 6.1.3.X.

* * * Second Change * * * *

6.6.1	General

This clause describes the security requirements and the security procedures that are specifically for 5G ProSe UE-to-UE Relay Communication defined in TS 23.304 [2].

The security requirements for 5G ProSe Layer-3 UE-to-UE Relay and 5G ProSe Layer-2 UE-to-UE Relay are defined in clause 6.6.2. The security procedures for 5G ProSe L3 UE-to-UE Relay and 5G ProSe Layer-2 UE-to-UE Relay are defined in clause 6.6.3 and clause 6.6.4 respectively. The security requirements and security procedures for 5G ProSe Multi-hop UE-to-Network Relay is defined in clause 6.3.Z.

* * * Third Change * * * *

6.1.3.4.1	General

This clause describes the security requirements and the procedures for 5G ProSe multi-hop UE-to-Network Relay Discovery defined in TS 23.304 [2],

* * * Fourth Change * * * *

6.1.3.4.2	Security requirements for 5G ProSe Multi-hop UE-to-Network Relay Discovery

The following security requirements apply to 5G ProSe multi-hop UE-to-Network Relay:

-	The 5G System shall support confidentiality protection, integrity protection, and replay protection of discovery messages for multi-hop UE-to-Network Relay discovery.

-	The 5G System shall provide means for mitigating trackability and linkability attacks on UEs during in multi-hop UE-to-Network Relay discovery.

-	The 5G System shall provide a means to securely provision the security materials for multi-hop UE-to-Network Relay discovery.

* * * Fifth Change * * * *

6.3.8	Security for 5G ProSe Communication via 5G ProSe Multi-hop UE-to-Network Relay

This clause describes the security requirements and the procedures for 5G ProSe multi-hop UE-to-Network Relay communication defined in TS 23.304 [2].

* * * Sixth Change * * * *

6.3.8.1	Security requirements

The following security requirements apply to 5G ProSe multi-hop UE-to-Network Relay:

-	The 5G System shall support the authorization and authorisation of the UEs involving in the 5G ProSe multi-hop UE-to-Network Relay communication.

-	The 5G System shall support confidentiality protection, integrity protection, and replay protection of the messages in the 5G ProSe multi-hop UE-to-Network Relay communication sceanrio.

-	The 5G System shall provide means for mitigating trackability and linkability attacks on UEs involving in the 5G ProSe multi-hop UE-to-Network Relay communication.

-	The 5G System shall provide a means to securely provision the security materials for 5G ProSe multi-hop UE-to-Network Relay communication.

* * * End of Changes * * * *