3GPP TSG-SA3 Meeting #120	S3-250717

# Athens, Greece, 17 - 21 February 2025

Source:	Sony

Title:	KI#5, Updated Conclusions

Document for:	Approval

Agenda item:	5.9

Spec:	3GPP TR 33.713

Version:	0.6.0

Work Item:	FS_AIOT_Sec

Comments

The Ambient IoT Check point at TSG SA#106 resulted in Way Forward in SP-241980.

The Way Forward includes important aspects on how to focus the work in Q1 to reach conclusions that can be implemented during the normative phase during Q2 and Q3, 2025.

The most important guidance is to focus the work on device type 1 (the most constrained device type) and topology 1 (base station reader), as this is what RAN will specify in Rel-19 according to approved WID in RP-243326.

During SA3#119adhoc e-meeting, KI#5 was discussed, and initial conclusions were agreed and captured in the TR. It was also clear that companies had different opinions. Some companies had the option that before any Ambient IoT service can be provided entity authentication must be performed, the network authenticates the AIoT device and the AIoT device authenticates the network. A different opinion was that the AIoT device and network entity authentication can be done as part of the Ambient IoT service provided.

As Rel-19 will only support the most constrained device types, the overhead should be minimized as much as possible. If entity (device/network) authentication can be performed during the provided Ambient IoT service as part of the message exchange, then that would result in reduced overhead. The question is whether such optimization can address the threats identified and the requirement captured in KI#5.

Observation: KI#5 does not explicitly require that the mutual authentication must be done before providing the Ambient AIoT service. Meaning that the authentication could be performed when providing the Ambient AIoT service.

Proposal 1: Capture in the conclusion that mutual authentication for Inventory + Command service is achieved by embedding the challenge and response into the service message flow.

Observation: The threats identified in KI#5 are 1) a fake AIoT device impersonate a legitimate AIoT device and 2) an adversary can impersonate a legitimate network.

An alternative and a simplification to the challenge and response mechanism captured in the initial conclusions can be to use the message protection discussed in KI#4. If a message from a AIoT device or network includes a MAC and the AIoT device identification is protected, then that can provide a solution with reduced complexity for device type 1 as message protection and identity protection are expected to be supported. Such solution would reduce the complexity and overhead when preventing fake devices or adversaries to impersonate legitimate entities.

Proposal 2: Capture in the conclusion that mutual authentication for Inventory + Command service can alternatively be achieved by adding MAC to the Command request and Command response.

Proposal 3: Capture in the conclusion that the authentication for Inventory service is achieved when the AIoT device responds with a protected ID never sent over the radio interface and optionally adding a MAC to the response.

Proposed Changes

* * * First Change * * * *

## 7.5	Conclusion on KI #5

Rel-19 only supports device type 1. To minimize protocol overhead, the entity (device/network) authentication shall be done during the Ambient IoT service as part of the message exchange.

1. 	The credential (including device ID and authentication credential) for AIoT device authentication can be owned by an operator or by a third party. The principles for device ID management are concluded in 8.2.1 in TR 23.700-13 [4].

2. 	For operator managed AIoT devices, the following principles are taken as the conclusion for KI#5:

Inventory-only Procedure:

- Authentication (i.e., at least one-way authentication; the 5G network authenticates the AIoT device) shall be supported.

- The authentication procedure is based on a MAC and protected ID.

-	The network includes a protected ID or a group trigger in the Inventory request message.

-	The AIoT device includes a protected ID and a MAC in the Inventory response. The AIoT device can authenticate the network if the Inventory Request included an expected protected ID. The network can always authenticate the AIoT device by checking the MAC.

NOTE X: Further details of the authentication procedure e.g., parameter(s) used for the 5G network to generate MAC and/or protected ID of the AIoT device are to be resolved during normative phase.

Inventory and Command Procedure:

- Mutual authentication between the AIoT device and the 5G network shall be supported.

- The authentication procedure is based on a challenge-response mechanism.

-	The network includes a challenge in the Inventory request message.

NOTE X: Adding a challenge in the Inventory request message depends of feedback from RAN2.

-	The AIoT device includes the response and a challenge to the network in the Inventory response

-	The network includes a response and the command in the Command request message if the AIoT device is authenticated.

-	The AIoT device handles the Command request if the network is authenticated.

NOTE X: Further details of the mutual authentication procedure and details of the challenge-response mechanism(e.g., parameter(s) used for the 5G network to authenticate the AIoT device and for the AIoT device to authenticate the network) are to be resolved during normative phase.

Alternatively

- The mutual authentication procedure is based on MAC and protected ID.

-	The network includes a protected ID in the Inventory request message.

-	The AIoT device includes a protected ID and a MAC in the Inventory response. The AIoT device responds if the network is authenticated, which is done by checking that the received protected ID is the expected protected ID.

-	The network includes a MAC and the command in the Command request message if the AIoT is authenticated. The network authenticates the AIoT device by checking the MAC received in the Inventory response message.

-	The AIoT device handles the Command request if the MAC is verified.

NOTE X: Further details of the authentication procedure e.g., parameter(s) used for the 5G network to generate MAC and/or protected ID of the AIoT device are to be resolved during normative phase.

3. 	Additional considerations during normative phase:

NOTE 3:	Where to store the AIoT device credentials and related policy information will be based on SA2 and SA3 coordination.

* * * End of Changes * * * *