3GPP TSG-SA3 Meeting #120	S3-250730

Athens, Greece, 17 - 21 February 2025

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.516 | CR | draftCR | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add test case TC_AUSF_CONFIRMATION_WITH_SUPI that check whether the AUSF is vulnerable to a confirmation attack |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | BSI |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | SCAS_5G_Maint |  |  |  |  |  | Date: |  |  | 2025-02-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | When a specific implementation decision is made (as seen in some open source implementations), the AUSF might be vulnerable to a confirmation attack when exposed to a rogue SEAF, e.g in a roaming scenario. This change requests adds a test case that verifies whether the AUSF is vulnerable to such an attack. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added a test case that checks if the AUSF is vulnerable to a confirmation attack from a rogue SEAF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If the test case in this change request is not included in the tests for the AUSF, the AUSF might be left vulnerable to a confirmation attack. The confirmation attack can be used to link a SUCI to a SUPI as a rogue SEAF in a roaming scenario. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.2.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TR 33.926 CR 0103 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

********** START OF 1st CHANGE **********

##### 4.2.2.X	Confirmation of Auth event with SUPI

Requirement Name: No linkability between SUCI and SUPI in AUSF confirmation

Requirement Reference: TS 33.501, clauses 6.1.2 and 6.1.3.2.0, and TS 29.509, clause 5.2.2.2.2.

Requirement Description:

As specified in TS 33.501, clause 6.1.2, the SEAF initiates an authentication request with the AUSF by using the SUCI or SUPI. As specified in TS 29.509, clause 5.2.2.2.2, The AUSF generates a sub-resource “5g-aka-confirmation” on which the SEAF shall send the confirmation for the authentication. The sub-resource link shall be provided by the AUSF to the SEAF in the response of the request.

As specified in TS 33.501, clause 6.1.3.2.0, the AUSF proceeds with step 11 in Figure 6.1.3.2-1 after receiving a Nausf_UEAuthentication_Authenticate request message from the SEAF. The AUSF should deny the authentication if RES* is not equal to HRES*.

Threat Reference: TR 33.926, clause F.2.2.2, Usage of SUCI or SUPI as authCtxId

Test case:

Test Name: TC_AUSF_CONFIRMATION_WITH_SUPI

Purpose:

Verify that the AUSF generates a secure context ID for the sub-resource that can never link a SUCI to a SUPI.

Pre-Conditions:

-	AUSF network product is connected in simulated/real network environment including a UDM and AMF. The UDM and AMF may be simulated.

-	The tester has access to the user credentials.

-	The SUPI is registered at the UDR.

Execution Steps:

- The tester triggers an Nausf_UEAuthentication request message sent from AMF to AUSF over the N12 interface containing the SUCI derived from the SUPI.

- The tester captures the Nausf_UEAuthentication response message sent from AUSF to AMF over the N12 interface containing HXRES*, AUTN and RAND values.

- The tester computes the HRES* value using the RAND value.

- The tester triggers an Nausf_UEAuthentication_Authenticate request message sent from AMF to AUSF over the N12 interface containing the computed HRES* value and using the SUPI as path parameter “authCtxId” (context ID) in the request.

- The tester captures the Nausf_UEAuthentication_Authenticate response message sent from AUSF to AMF over the N12 interface.

Expected Results:

The status code of the Nausf_UEAuthentication_Authenticate response message sent from AUSF to AMF is 404 (not found).

Expected format of evidence:

Evidence suitable for the interface, e.g., evidence can be presented in the form of log messages or a packet trace. A packet trace should at least contain the messages sent between the AMF and AUSF on the N12 interface.

********** END OF CHANGE **********