3GPP TSG-SA3 Meeting #120	S3-251026

Athens, Greece, 17 - 21 February 2025	revision of S3-250599

# Source:	Ericsson

# Title:	Moving CIBA discussion from KI#1.2 to KI#3

# Document for:	Approval

Agenda Item:	5.19

# 1	Decision/action requested

It is proposed to add the following update to the TR 33.700-22.

# 2	References

[1]	3GPP TR 33.700-22: "Study on security aspects of CAPIF Phase3".

# 3	Rationale

It is proposed to move the CIBA flow related discussions from KI#1.2 to KI#3.

# 4	Detailed proposal

It is proposed to approve the following change to TR 33.700-22 [1].

***START OF THE CHANGE***

### 5.1.2	Key Issue #1.2: Resource owner authorization management

#### 5.1.2.1	Key issue details

KI#1 of TR 23.700-22 [3] is studying resource owner authorization management (e.g., authorizing access to the resource owner's resource or revoking the authorization of access to the resource owner's resource). There is a NOTE in TR 23.700-22 [3]:

NOTE:	Aspects pertaining to the definition of resource owner consent/authorization over CAPIF-8 are in the scope of SA3, noting that the R18 security aspects of CAPIF supporting RNAA are specified in 3GPP TS 33.122 [4].

This key issue studies how to authenticate and authorize the resource owner to provide resource owner authorization.

#### 5.1.2.2	Security threats

Without the authentication between resource owner and authorization server, malicious resource owner can impersonate victim resource owner to do resource owner authorization management.

#### 5.1.2.3	Potential Security Requirement

Mutual authentication between the authorization server and the resource owner should be supported.

CAPIF RNAA should support to authorize the resource owner to provide resource owner authorization.

CAPIF RNAA should support authorization of API invoker based on resource owner authorization and should support revocation of the resource owner authorization.

***NEXT CHANGE***

## 5.3	Key Issue #3: Authorizing API invoker on one UE accessing resources related to another UE

### 5.3.1	Key issue details

This key issue addresses the security aspects of 23.700-22 KI #6 [3].

It studies the security aspects for the case that API invoker(s) are deployed on one UE and requests to access resources (hosted in the network) related to another UE (e.g., application client on UE is fetching location of another UE or setting QoS for PDU sessions of another UE).

As specified in 3GPP TS 23.222 [2], the API invoker may be deployed in any of the following ways:

a.	API invoker may be deployed as AF on the UE (i.e. 3rd party application).

b.	API invoker may be deployed as AF on the UE supporting several other 3rd party applications deployed on the UE.

c.	API invoker may be deployed on the network as AF.

So far, only a UE accessing its own resources is considered if the API invoker is on a UE. Resource owner-aware northbound API access (RNAA) defined in TS 33.122 [4] only supports authorizing API invoker on one UE to request resources related to the same UE.

Therefore, it is proposed to study how to authorize an API invoker on one UE to access resources related to another UE.

The use case in Annex A.2 of TR 23.700-95 [13] is UE-originated API invocation (Location tracking) where the application running on a UE initiates the procedure and the resource owner can be not using the application at the API invocation time. In CAPIF RNAA, only client credential and authorization code flow (w/o PKCE) are supported. In the client credential flow, it is assumed that the resource owner authorization information is available. In the authorization code flow, it is assumed that the client (API invoker) can access to the user agent of the resource owner and direct the resource owner to the authorization server.

### 5.3.2	Security threats

RNAA only supports authorizing API invoker on one UE to request resources related to the same UE. Hence, the CAPIF can only select non-RNAA (i.e., authorization procedure without resource owner involvement) based authorization mechanism for API invoker on one UE request to access resources related to another UE. This may lead to information leakage of the resource owner if the API invoker is authorized without engaging of the resource owner.

Without a proper security mechanism, unauthorized API invokers can access to resources related to a UE, which potentially results in sensitive information leakage and unauthorized modification to the resources accessed by northbound APIs.

### 5.3.3	Potential security requirements

CAPIF should support a mechanism for authorization of the API invoker on one UE to access resources related to another UE.

***NEXT CHANGE***

## 6.0	Mapping of solutions to key issues

Table 6.0-1: Mapping of solutions to key issues

| Solutions | KI#1 | KI#1.1 | KI#1.2 | KI#1.3 | KI#2 | KI#3 | KI#4 | KI#5 | KI#6 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Solution #1: Security protection mechanism for CAPIF-8 reference point |  | X |  |  |  |  |  |  |  |
| Solution #2: CAPIF-8 reference point security |  | X |  |  |  |  |  |  |  |
| Solution #3: Security procedures for CAPIF-8 reference points |  | X |  |  |  |  |  |  |  |
| Solution #4: resource owner authorized revocation |  |  | X |  |  |  |  |  |  |
| Solution #5: Security of resource owner authorization management and CAPIF-8 | X |  |  |  |  |  |  |  |  |
| Solution #6: Security procedures for resource owner authorization management |  |  | X |  |  |  |  |  |  |
| Solution #7: RO permission/authorization management |  |  | X |  |  |  |  |  |  |
| Solution #8: Resource owner triggered revocation procedure |  |  | X |  |  |  |  |  |  |
| Solution #9: Resource owner authentication and authorization mechanism |  |  | X |  |  |  |  |  |  |
| Solution #10: resource-level and/or api-level authorization and revocation |  |  |  | X |  |  |  |  |  |
| Solution #11: Client initiated backchannel authorization (CIBA) based solution |  |  |  |  |  | X |  |  |  |
| Solution #12: Security method retrieval in CAPIF interconnect |  |  |  |  | X |  |  |  |  |
| Solution #13: Requesting security information from another CCF in order to authenticate using TLS-PSK in CAPIF interconnect |  |  |  |  | X |  |  |  |  |
| Solution #14: Authentication aspect in CAPIF interconnect when API invoker has not included CCF information |  |  |  |  | X |  |  |  |  |
| Solution #15: Authorization token request handling in CAPIF interconnect |  |  |  |  | X |  |  |  |  |
| Solution #16: Mapping an API invoker authorization request to the correct CCF in CAPIF interconnect |  |  |  |  | X |  |  |  |  |
| Solution #17: Security procedures for CAPIF interconnection |  |  |  |  | X |  |  |  |  |
| Solution #18: API invoker authentication mechanism in CAPIF interconnection scenarios |  |  |  |  | X |  |  |  |  |
| Solution #19: API invoker authorization mechanism in CAPIF interconnection scenarios |  |  |  |  | X |  |  |  |  |
| Solution #20: Security method negotiation mechanism in CAPIF interconnection scenarios |  |  |  |  | X |  |  |  |  |
| Solution #21: Solution for CAPIF interconnection security |  |  |  |  | X |  |  |  |  |
| Solution #22: CAPIF interconnection |  |  |  |  | X |  |  |  |  |
| Solution #23: Security protection mechanism for CAPIF-6 and CAPIF-6e reference points |  |  |  |  | X |  |  |  |  |
| Solution #24: Security procedure for CAPIF interconnection |  |  |  |  | X |  |  |  |  |
| Solution #25: Backend based solution for UE-deployed API invoker accessing resources not owned by that UE |  |  |  |  |  | X |  |  |  |
| Solution #26: Nested API invocation |  |  |  |  |  |  | X |  |  |
| Solution #27: Authorization for nested API invocation |  |  |  |  |  |  | X |  |  |
| Solution #28: Authenticating multiple API invokers of the same RO |  |  |  |  |  |  |  | X |  |
| Solution #29: Enhancing authorization through finer granularity access token |  |  |  | X |  |  |  |  |  |
| Solution #30: Authentication of the origin API invoker in nested API invocation |  |  |  |  |  |  | X |  |  |
| Solution #31: Authorization mechanism for nested API invocation |  |  |  |  |  |  | X |  |  |
| Solution #32: Validation of correct GPSI in API invoker information |  |  |  |  |  |  |  | X |  |
| Solution #33: Onboarding of API Invoker residing in UE |  |  |  |  |  |  |  |  | X |
| Solution #34: UE-deployed API invoker accessing resources not owned by that UE | X |  |  |  |  | X |  |  |  |

***NEXT CHANGE***

## 6.11	Solution #11: Client initiated backchannel authorization (CIBA) based solution

### 6.11.1 	Introduction

This solution addresses the requirement identified in key issue #3 (Authorizing API invoker on one UE accessing resources related to another UE ).

More precisely, this solution addresses the following requirements by allowing offline interaction with the resource owner after triggering of API invocation by the API invoker.

In some cases, there can be no preconfigured authorization information and at the API invocation time the resource owner is not using the application which needs to direct the resource owner to the authorization server. UE-originated API invocation (Location tracking) captured in Annex A.2 of TR 23.700-95 [13] can be given as an example use case. Client credentials (requiring a preconfigured authorization data) and authorization code flow (including PKCE) (requiring resource owner accessible by the API invoker at the API invocation time) are not enough to address these type of use cases. Utilization of client-initiated backchannel authorization (CIBA) [11] flow is proposed for such use cases.

### 6.11.2	Solution details

High-level steps of the solution are presented in Figure 6.11.2-1 and explained below. For details the CIBA flow [11] can be checked.

Figure 6.11.2-1: High-level procedure flow of the solution

1.	The AF sends the authorization request to the CCF.

2.	The CCF/Authorization Function checks whether the request requires resource owner permission. If the request requires resource owner permission and there is no available resource owner permission data, then the CCF/Authorization Function decides to return ACK to the AF.

3.	The CCF/Authorization Function returns ACK to the AF, so that the AF executes steps 5 and 6 regularly until the CCF/Authorization Function obtains permission from the resource owner.

4.	The CCFAuthorization Function obtains permission from the resource owner via ROF. Note that this is an offline step, i.e., the execution of steps 5-6 does not have any impact on this step.

5-6. The AF regularly sends the token request to the CCF/Authorization Function until the CCF/Authorization Function obtains the permission from the resource owner. The CCF/Authorization Function sends the response which depends on whether the CCF/Authorization Function has obtained the permission from the resource owner.

### 6.11.3	Evaluation

The solution addresses the use cases where the API invoker and resource owner are residing on different UEs. The CCF/Authorization Function needs to support the CIBA mechanism. There is no impact on the AEF.

Steps 5-6 are executed multiple times between the API invoker and CCF/Authorization Function. These steps are existing steps in the CIBA specification.

The CIBA based solution especially is for the case that the API invoker and the resource owner are residing on different devices.

***END OF CHANGES***