3GPP TSG-SA3 Meeting #120	S3-251072

# Athens, Greece, 17 - 21 February 2025													revision of S3-250441

Source:	Qualcomm Incorporated

Title:	Pseudo-CR on 3GPP Cryptographic Inventory for DTLS and TLS

Document for:	Approval

Agenda item:	5.20

Spec:	3GPP TR 33.9xy

Version:	0.0.0

Work Item:	FS_CryptoInv

Comments

This document presents an inventory of interfaces/protocols that make use of DTLS and TLS protocols in response to the work item Study on 3GPP Cryptographic Inventory. Only uses of DTLS and TLS that refer to security profiles in 3GPP specifications are included.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[aa]	3GPP TS 33.210: "3G security; Network Domain Security (NDS); IP network layer security".

[ab]	3GPP TS 33.310: "Network Domain Security (NDS); Authentication Framework (AF)".

[ac]	3GPP TS 33.501: “Security architecture and procedures for 5G system”.

* * * Next Change * * * *

# 4.4	Detailed Protocol List

## 4.4.1		DTLS

DTLS is used in 5G system in standalone mode to protect the following:

N2 interface (see clause 9.2 of TS 33.501 [ac]).

Xn interface (see clause 9.4 of TS 33.501 [ac]).

DIAMETER or GTP-based interfaces (see clause 9.5 of TS 33.501 [ac]).

gNB internal interfaces (see clause 9.8 of TS 33.501 [ac]).

Security profiles for DTLS implementation and usage in 3GPP are given in clause 6.2 of TS 33.210 [aa] and the certificate profile is given in clause 6.1.3a of TS 33.310 [ab].

DTLS employs symmetric cryptography for confidentiality and integrity protection.

DTLS employs asymmetric cryptography for digital signature and key agreement.

## 4.4.2	TLS

TLS is used in 5G system in standalone mode to protect the following:

NIDD interfaces (see clause 6.16.3 of TS 33.501 [ac]).

DIAMETER or GTP-based interfaces (see clause 9.5 of TS 33.501 [ac]).

NEF – AF interface (see clauses 12.2 and 12.3 of TS 33.501 [ac]).

Interfaces between network functions (see clauses 13.1, 13.2, 13.5 of TS 33.501 [ac]).

N32 interface (see clause 13.2 of TS 33.501 [ac]).

Network slice management interfaces (see clauses 15.2 and 15.3 of TS 33.501 [ac]).

Message Service interfaces for MIoT over the 5G System (see clauses Y.2 – Y.4 of TS 33.501 [ac]).

Security profiles for TLS implementation and usage in 3GPP are given in clause 6.2 of TS 33.210 [bb] and the certificate profile is given in clause 6.1.3a of TS 33.310 [cc].

TLS employs symmetric cryptography for confidentiality and integrity protection.

TLS employs asymmetric cryptography for digital signature and key agreement.

* * * End of Changes * * * *