# 3GPP TSG-SA3 Meeting #120	S3-251124

# Athens, Greece, 17 - 21 February 2024

Title:	Reply LS on security handling for inter-CU LTM in non-DC cases

Response to:	S3-250325/R2-2411136 Reply LS on security handling for inter-CU LTM in non-DC cases

Release:	Rel-19

Work Item:	NR_Mob_Ph4-Core

Source:	SA3

To:	RAN2, RAN3

Cc:	-

Contact Person:	Wei Zhou

E-mail Address:	zhouwei@catt.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

SA3 thanks RAN2 for the LS on the security handling for inter-CU LTM in non-DC cases.

Considering the RAN2’s request on avoiding RRC configuration between cell switches for subsequent inter-CU LTM in solution selection, SA3 reached a compromise that the NCC can be transmitted to the UE in plaintext via MAC CE message, even though the unprotected MAC leads to increased attack scenarios and probability of handover failure.

In addition, SA3 will not consider the protection of MAC CE messages in this release, but may consider studying MAC CE protection in the future.

The details on key distribution between the source gNB and the target gNB are left to RAN3 to decide and inform SA3. RAN3 should consider the scenario where the serving gNB utilizes the unused NH (if any is received from a path switch procedure at execution phase) to derive the KNG-RAN* for LTM handover.

2. Actions:

To RAN2 and RAN3

ACTION: SA3 kindly asks RAN2 and RAN3 to take the above information into account.

To RAN3

ACTION: SA3 kindly asks RAN3 to decide and update the SA3 on the key distribution mechanism.

3. Date of Next SA3 Meeting:

SA3#121			April 07th – 11st, 2025			Goteborg, SE

SA3#122			May 19st – 23th, 2025			Fukuoka, JP