3GPP TSG-SA3 Meeting #122	S3-251939

# Fukuoka, Japan,  19 - 23 May 2025

Source:	ZTE

Title:	Update Security mechanism and procedures for inter-CU LTM

Document for:	Approval

Agenda item:	4.12

Spec:

Version:

Work Item:	Security aspects of NR mobility enhancement Phase 4

Comments

This pCR proposes to update the draft CR. According to the response from RAN3 (R3-252449), the new source gNB provides the KgNB*(s) to the corresponding candidate gNB via a LTM Configuration Update procedure. Therefore, it is proposed to update the relevant clause accordingly.

Proposed Changes

* * * First Change * * * *

6.X.Y	Security mechanism and procedures for inter-CU LTM

In the case where CU is acting as SN and MN is unchanged, the security mechanism and procedures for SCPAC as specified in clause 6.10.2.4 apply.

In the case where CU is acting as MN and DC is not configured, the source gNB shall derive a KNG-RAN* using the target PCI, target ARFCN-DL/EARFCN-DL and the KgNB/NH based on either a horizontal key derivation or a vertical key derivation as described in Annex A.11/Annex A.12. The source gNB shall forward the { KNG-RAN*, NCC} pair to the target gNB. The target gNB uses the received KNG-RAN* directly as KgNB to be used with the UE. The target gNB associates the NCC value received from source gNB with the KgNB. The source gNB includes the NCC in plain text into the cell switch command MAC CE and sends to UE. Upon receiving the cell switch command message,  if the NCC of the current KgNB is less than the received NCC value, the UE first synchronize the locally kept NH parameter iteratively and increasing the NCC value until it matches the NCC.  If the received NCC value is equal to the NCC value associated with the current KgNB, then the UE uses the active KgNB to derive the KNG-RAN*. After each handover, the new source gNB provides the new KgNB* to the corresponding candidate gNB via a LTM Configuration Update procedure.

If the gNB receives mismatched UE security context (e.g. UE’s 5G security capabilities and/or UE’s UP security policy) in the Path-Switch Acknowledge message after LTM switch, the gNB initiates an intra-cell handover procedure which includes RRC Connection Reconfiguration procedure indicating the change of selected algorithms (as defined in clause 6.7.3.1) and/or UE’s UP activation status (as defined in clause 6.6.1) to the UE. The gNB also updates the UE’s 5G security capabilities and/or UE’s UP security policy for the active PDU session to all candidate gNBs.

* * * End of Changes * * * *