# 3GPP TSG-SA3 Meeting #122																			S3-252344

# Fukuoka, Japan, 19 – 23 May 2025

Source:	Qualcomm Incorporated

Title:	Pseudo-CR on 3GPP Cryptographic Inventory Table for TLS

Document for:	Approval

Agenda item:	5.20

Spec:	3GPP TR 33.938

Version:	0.2.0

Work Item:	FS_CryptoInv

Comments

This document provides entries for tables 4.2-1 and 4.3-1 corresponding to clause 4.4.2 TLS.

The profiles of TLS 1.2 and TLS 1.3 are presented separately in TS 33.210 and are different from each other: clause 6.2.2 provides the profile of TLS 1.3 and clause 6.2.3 provides the profile of TLS 1.2. Accordingly, we have created separate table entries of TLS 1.2 and TLS 1.3. Here is a brief analysis of how the table entries were determined.

For TLS 1.2, RFC 5246 clause 9 (pp. 65) says “In the absence of an application profile standard specifying otherwise, a TLS-compliant application MUST implement the cipher suite TLS_RSA_WITH_AES_128_CBC_SHA (see Appendix A.5 for the definition).”

However, since TS 33.210 clause 6.2.3 says “Only cipher suites with AEAD (e.g. GCM) and PFS (i.e., ECDHE) shall be supported.”, the cipher suite TLS_RSA_WITH_AES_128_CBC_SHA is not supported in 3GPP.

Other cipher suites mentioned (either mandatory or recommended) in TS 33.210 clause 6.2.3 are:

- TLS_ECDHE_ECDSA_WITH_AES_128_GCM_SHA256

- TLS_DHE_RSA_WITH_AES_128_GCM_SHA256

- TLS_ECDHE_RSA_WITH_AES_128_GCM_SHA256

- TLS_ECDHE_ECDSA_WITH_AES_256_GCM_SHA384

- TLS_ECDHE_RSA_WITH_AES_256_GCM_SHA384

However, since TS 33.210 clause 6.2.3 says “Finite field Diffie-Hellman (i.e. DHE) shall not be supported”, the cipher suite TLS_DHE_RSA_WITH_AES_128_GCM_SHA256 is also not supported in 3GPP. So, the only 4 cipher suites in bold above are supported in 3GPP. In summary, the supported cryptographic algorithms for TLS 1.2 are:

- Asymmetric:

  - Key agreement: ECDHE

  - Digital signature: ECDSA, RSA

- Symmetric:

  - Authenticated encryption: AES_128_GCM, AES_256_GCM

  - Hash: SHA256, SHA384

Similarly, for TLS 1.3 RFC 8446 clause 9.1 (pp. 102) says “A TLS-compliant application MUST implement the TLS_AES_128_GCM_SHA256  [GCM] cipher suite and SHOULD implement the TLS_AES_256_GCM_SHA384 [GCM] and TLS_CHACHA20_POLY1305_SHA256 [RFC8439] cipher suites (see Appendix B.4). A TLS-compliant application MUST support digital signatures with rsa_pkcs1_sha256 (for certificates), rsa_pss_rsae_sha256 (for CertificateVerify and certificates), and ecdsa_secp256r1_sha256. A TLS-compliant application MUST support key exchange with secp256r1 (NIST P-256) and SHOULD support key exchange with X25519 [RFC7748].”

So, the supported cryptographic algorithms for TLS 1.3 are:

- Asymmetric:

  - Key agreement: ECDHE

  - Digital signature: ECDSA, RSA

- Symmetric:

  - Authenticated encryption: AES_128_GCM, AES_256_GCM, CHACHA20_POLY1305

  - Hash: SHA256, SHA384

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 33.210: "3G security; Network Domain Security (NDS); IP network layer security".

[3]	3GPP TS 33.310: "Network Domain Security (NDS); Authentication Framework (AF)".

[4]	3GPP TS 33.501: “Security architecture and procedures for 5G system”.

[5]	IETF RFC 9190: "EAP-TLS 1.3: Using the Extensible Authentication Protocol with TLS 1.3".

[6]	IETF RFC 5216: "The EAP-TLS Authentication Protocol",

[7]	SECG SEC 1: “Recommended Elliptic Curve Cryptography”, Version 2.0, 2009. Available at http://www.secg.org/sec1-v2.pdf.

[8]	SECG SEC 2: “Recommended Elliptic Curve Domain Parameters”, Version 2.0, 2010. Available at http://www.secg.org/sec2-v2.pdf.

[9]	IETF RFC 9001: "Using TLS to Secure QUIC".

[10]	IETF RFC 8152: "CBOR Object Signing and Encryption (COSE)".

[11]	3GPP TS 33.220: “Generic Authentication Architecture (GAA); Generic Bootstrapping Architecture (GBA)”.

[12]	IETF RFC 8613: "Object Security for Constrained RESTful Environments (OSCORE)".

[13]	3GPP TS 33.180: "Security of the Mission Critical (MC) service".

[14]	IETF RFC 6509: ''MIKEY-SAKKE: Sakai-Kasahara Key Encryption in Multimedia Internet KEYing (MIKEY)''.

[15]	IETF RFC 5448: "Improved Extensible Authentication Protocol Method for 3rd Generation Authentication and Key Agreement (EAP-AKA')".

[16]	3GPP TS 35.205: "3G Security; Specification of the MILENAGE algorithm set: An example algorithm set for the 3GPP authentication and key generation functions f1, f1*, f2, f3, f4, f5 and f5*".

[17]	3GPP TS 35.231: "Specification of the TUAK algorithm set: A second example algorithm set for the 3GPP authentication and key generation functions f1, f1*, f2, f3, f4, f5 and f5*; Document 1: Algorithm specification".

[18]	3GPP TS 35.234: "Specification of the MILENAGE-256 algorithm set; An example set of 256-bit 3GPP authentication and key generation functions f1, f1*, f2, f3, f4, f5, f5* and f5**; Document 1: General".

[aa]	IETF RFC 5246: “The Transport Layer Security (TLS) Protocol Version 1.2”.

[ab]	IETF RFC 8446: “The Transport Layer Security (TLS) Protocol Version 1.3”.

* * * Next Change * * * *

Table 4.2-1: Protocols Used in 3GPP Employing Symmetric Cryptographic Algorithms (5G System)

| Protocol/Function | Protocol Profile, Clauses | Cryptographic Algorithm(s) | Feature(s), Usage Type |
| --- | --- | --- | --- |
| TLS 1.2 (IETF RFC 5246 [aa]) | TS 33.210 [2] clauses 6.2.1, 6.2.3 | AES_128_GCM, AES_256_GCM | Confidentiality and Integrity Protection |
|  |  | SHA256, SHA384 | Hash Function |
| TLS 1.3 (IETF RFC 8446 [ab]) | TS 33.210 [2] clauses 6.2.1, 6.2.2 | AES_128_GCM, AES_256_GCM, CHACHA20_POLY1305 | Confidentiality and Integrity Protection |
|  |  | SHA256, SHA384 | Hash Function |

* * * Next Change * * * *

Table 4.3-1: Protocols Used in 3GPP Employing Asymmetric Cryptographic Algorithms (5G System)

| Protocol/Function | Protocol Profile, Clauses | Cryptographic Algorithm(s) | Feature(s), Usage Type |
| --- | --- | --- | --- |
| TLS 1.2 (IETF RFC 5246 [aa]) | TS 33.210 [2] clauses 6.2.1, 6.2.3 | ECDHE | Key Agreement |
|  |  | ECDSA, RSA | Digital Signature |
| TLS 1.3 (IETF RFC 8446 [ab]) | TS 33.210 [2] clauses 6.2.1, 6.2.2 | ECDHE | Key Agreement |
|  |  | ECDSA, RSA | Digital Signature |

* * * End of Changes * * * *