3GPP TSG-SA3 Meeting #122																			S3-252145

Fukuoka, Japan, 19 – 23 May 2025

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.210 | CR | 0086 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New JWS profiles for CCA tokens and access tokens |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2025-05-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Some existing JWS profile requirements are applicable only to the N32 interface. Therefore, it is not necessary to make those requirements applicable for all 3GPP entities and functions that support JWS. TS 33.501, in Clause 13.3.8.2, specifies that CCA shall include either x5u to refer to a resource for the X.509 public key certificate or certificate chain, or x5c to include the X.509 public key certificate or certificate chain used for signing the client authentication assertion. However, JWS profile does not specify the use of JOSE header for the CCA and access token. The message carrying an access token also is expected to use JOSE header parameter kid and x5t#S256 to identify the raw public key or X.509 certificate required to verify the signature in the access token. The JWS profile makes kid mandatory to be supported by all 3GPP entities and functions that support JWS according to RFC 7515. However, RFC 7515 does not explicitly specify kid to be used to identify a certificate or certificate chain. An identifier that is meant for identifying a certificate or certificate chain would be more appropriate. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Separate JWS profile requirement for N32 interface, CCA toekens, and access tokens. Adding x5u and x5c to CCA profile requirements Adding x5t#S256 to access token profile requirements |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** BEGIN OF CHANGE ***

### 6.3.3	JWS profile

#### 6.3.3.1	General

All entities and functions that support JWS according to RFC 7515 [46] shall follow the following restrictions and extensions:

The "jwk" header parameter shall not be supported.

The "alg" header parameter value ES256 (ECDSA using P-256 and SHA-256) shall be supported.

The "none" "alg" header parameter value shall not be supported.

The "kid" header parameter shall be supported.

If the "kid" header parameter is used, the end point shall check the indicated "alg" matches the one specified by the parameters.

#### 6.3.3.2	N32 profile requirements

End points may establish the expected signing algorithm and associated keys out-of-band (e.g. N32-c) and use the "kid" header parameter to pass a key identifier.

If an end point has established a public key and algorithm out of band (e.g. N32-c) and the "kid" header parameter is not used, then the end point shall check the indicated "alg" header parameter against the established algorithm.

6.3.3.3	CCA profile requirements

The "x5u" header parameter shall be supported.

The "x5c" header parameter shall be supported.

6.3.3.4	Access Token profile requirements

The "x5t#S256" header parameter shall be supported.

The HS256 algorithm may be used for MACs.

*** END OF CHANGE ***