3GPP TSG-SA3 Meeting #123	S3-252549

# Goteborg, Sweden, 25 – 29 August 2025

Source:	China Mobile, AT&T, Vodafone, T-Mobile US, NTT DOCOMO, BT, Verizon, Telecom Italia, Charter Communications, Deutsche Telekom, IIT Bombay, KDDI

Title:	pCR to TS 33.502 - Collection requirements for Security related Events

Document for:	Approval

Agenda item:	5.1.1

Spec:	3GPP TS 33.502

Version:	0.0.1

Work Item:	SECHAND

Comments

Based on approved WID (SP-250876), the working task needs to specify the general requirements for security events handling and collection.

In WT2, it will specify the events that need to be reported including the following.

- 	The NF collects information on the SBA layer about malformed messages it receives that deviate from the 3GPP specified messages or are considered invalid according to the protocol specification and network state.

- 	The NF collects information about events involving receiving a massive number of incoming messages on the SBA layer.

- 	The NF collects information about failed authentication and authorization attempts from inbound connections on the SBA layer.

- 	The NF collects information about potential replay attacks on the SBA layer.

- 	The NF collects information about potential abnormal SBI call flows as defined for the communication models in Annex E of TS 23.501.

It is proposed to add security requirements for collection of security related events.

* * * First Change * * * *

# 5	Requirements on Security related events

Editor’s Note: This clause addresses the general requirements on configuration, collection and communication of events.

## 5.x	Requirements on events collection

Security related events data shall be stored with integrity and confidentiality protection.  An isolated environment can be used to keep storage away from other data pertaining to the NF. The following list further defines the requirements of the isolated environment:

-	The environment shall support secure storage of Events data as well as any other data related to security events.

-	The environment shall support secure execution of sensitive functions over stored data, e.g. en-/decryption of events data and any step involving long term secrets.

-	The environment shall support necessary access control mechanisms.

* * * End of Changes * * * *