3GPP TSG-SA3 Meeting #123																			S3-252742

Goteborg, Sweden, 25 – 29 August 2025

# Source:	OPPO

# Title:	New Key issue on authorization for sensing service

# Document for:	approval

Agenda Item:	6.1.8

1	Decision/action requested

It is proposed to approve New Key issue on authorization for sensing service.

2	References

N/A

3	Rationale

It is proposed to approve the new key issue on authorization for sensing service.

4	Detailed proposal

* * * First Change * * * *

5.X	Key Issue #X: Authorization for sensing service

5.X.1	Key issue details

Sensing service utilizes the radio signals in a 3GPP system to acquire information about object and/or characteristics of the environment, which provide a broad range of new functionality, such as environment and motion monitoring, object detection and tracking. SA1 has identified the use cases and requirements for ISAC in Rel-19 in TS 22.137 [x], including the corresponding security and privacy requirements. Recently, SA2 is studying the architecture enhancement to support sensing service in TR 23700-14 [y], which emphasizes that “Privacy protection and other security aspects will be coordinated with SA WG3”.

Unauthorized access is an important security threat before executing sensing services. Without proper authorization, unauthorized entities, e.g., RAN and UE, will be able to participate in sensing service operations and provide fake sensing data. Furthermore, an unauthorized sensing service consumer, i.e., AF or client UE, can obtain the sensing results by triggering sensing service, which may also give to the privacy leakage of the UEs involved in the sensing service.

In addition, KI #2 in TR 23.700-14 [y] also describes the authorization issue on the support of sensing service:

-	authorization of Sensing Service requests and revocation of an on-going Sensing Service; and

-	authorization and revocation for the Sensing Entities if applicable.

Hence, the further details about authorization of sensing entities and sensing service consumer should be studied in SA3.

5.X.2	Security threats

An unauthorized entity can pretend to be a transmitter or receiver, which may result in inaccurate sensing result.

An unauthorized sensing service consumer can obtain the sensing result by triggering Sensing Service between the entities, which may violate the privacy of the UEs involved in the Sensing service.

5.X.3	Potential security requirements

The 3GPP system shall be able to support the authorization of the sensing entities as a transmitter or receiver in the Sensing service.

The 3GPP system shall be able to support the authorization of a sensing service consumer for triggering sensing service and obtaining the sensing results.

The 3GPP system shall support privacy protection of the UEs involved in the sensing service for sensing service exposure.

* * * Second Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

…

[x]	3GPP TS 22.137: "Service requirements for Integrated Sensing and Communication; Stage 1".

[y]	3GPP TR 23.700-14: " Study on Integrated Sensing and Communication; Stage 2".

* * * End of Changes * * * *