3GPP TSG-SA3 Meeting #123	S3-252871

# Goteborg, Sweden, 25 – 29 August 2025

Source:	Xiaomi

Title:	Key Issue on PSK Derivation

Document for:	Endorsement

Agenda item:	6.1.6

Spec:	3GPP TS/TR <TS/TR number>

Version:	<TS version>

Work Item:	<Work Item>

Comments

SA2 has specified MPQUIC in clause 5.32.6.2.2 of TS 23.501 for steering, switching and splitting of UDP traffic (in Rel-18) and non-UDP traffic (in Rel-19) between the UE and UPF. In Annex AA.2 of TS 33.501, it was specified that TLS is used to secure MPQUIC for ATSSS, for which the security mode used for TLS authentication and tunnel establishment (asymmetric certificate mode or symmetric PSK mode) is open. In the newly endorsed 5GA R20 SID for security aspects of QUIC/TLS documented in S3-252354, the only objective is to study key derivation and delivery for UPF to support PSK mode.

The pre-shared key between the UE and the UPF can be obtained in multiple manners, for example, through pre-configuration with hard-coded key or derivation from the security context between the UE and the network. In Annex AA.2 of TS 33.501, it was noted that hard-coded pre-shared keys need to be avoided.

Accordingly, this paper proposes to add a new key issue on pre-share key derivation for QUIC security study.

* * * First Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[x]	3GPP TS 23.501: "System Architecture for the 5G System".

[y]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

* * * Second Change * * * *

5.X	Key Issue #X: PSK derivation

5.X.1	Key issue details

MPQUIC is specified in clause 5.32.6.2.2 of TS 23.501 [x] for steering, switching and splitting of UDP traffic and non-UDP traffic between the UE and UPF. In Annex AA.2 of TS 33.501 [y], TLS is specified to be used to secure MPQUIC for ATSSS, for which the security mode used for TLS authentication and tunnel establishment (asymmetric certificate mode or symmetric PSK mode) is open.

The pre-shared key between the UE and UPF can be obtained in multiple manners, e.g. through pre-configuration with hard-coded key or derivation from the security context between the UE and network. In Annex AA.2 of TS 33.501 [y], it was noted that hard-coded pre-shared keys need to be avoided.

According to the scope of the present document, only the PSK mode is to be supported. This key issue addresses the PSK key derivation on both the UE side and network side.

5.X.2	Security threats

If the pre-share key cannot be derived on the UE side or the network side, the UE and the UPF operating on PSK mode are not able to authenticate each other and set up TLS tunnel for the communication over MPQUIC protocol.

If the key pre-shared between the UE or UPF is not derived in a secure manner, an attacker may be able to obtain the pre-shared key and impersonate either the UE or the UPF to set up TLS tunnel for the communication with the victim over MPQUIC protocol.

5.X.1	Potential security requirements

The 5G system shall be able to support secure derivation of a key pre-shared between the UE and UPF for TLS authentication and tunnel establishment.

* * * End of Changes * * * *