3GPP TSG SA WG3 (Security) Meeting #74-BIS	S3-140385

31 March – 2 April, 2014;Sophia-Antipolis (France)	revision of S3-14abcd

Source:	Alcatel Lucent

Title:	WebRTC security: pCR to TR 33.871

Document for:				Approval

Agenda Item:	4.5 WebRTC security

Work Item / Release:	IMS_WebRTC / Rel-12

Abstract of the contribution: This pCR updates REQ1 to handle the usecase of an unauthorized IMS subscriber gaining access to IMS via a breached 3rd party service (scenario 2)

- Introduction

This pCR applies to scenario #2 from TR 23.701where the IMS Service provider relies on a third party authentication service to authenticate the user behind the downloaded WebRTC IMS Client (WIC).  The third party authentication service performs authentication using the user’s web identity and provides access credentials (for e.g. a token) to the WIC. The WIC presents the access credentials to the eP-CSCF in a W2 control plane message. The eP-CSCF validates the access credentials, and provided the validation is successful, performs IMS registration on behalf of the user.

Following are some examples of security vulnerabilities that exist in the above scenario:

- The third party authentication service is compromised and provides IMS access credentials belonging to the unsuspecting IMS subscriber to other unscrupulous users. The impacted IMS subscriber cannot use IMS services until the situation is rectified. REQ1 in section 5.1 of TR 33.871 identify the scope of the impact due the breach while REQ2 in the same section isolates the impact of the breach without letting it spread to other clients who use a different third party authentication service.

- A breached third party authentication service that is not authorized to authenticate a particular IMS subscriber ends up wrongfully authenticating and providing IMS access credentials to that subscriber. The IMS subscriber presents it in a W2 REGISTER message to get unauthorized and possibly free access to the IMS network

A change is proposed in the existing REQ1 to address b).

*** BEGIN CHANGES ***

# 5.0 	Assumptions and Security requirements

## 5.1	Assumptions

Editor’s Note: If needed, this clause will define the underlying assumptions of the work.

## 5.2	Security requirements

Requirements for Support of WebRTC IMS Client access to IMS are specified by SA1 in 3GPP TS 22.228 [2]. Additional potential architectural requirements identified by SA2 are stated in 3GPP TR 23.701 [5].

The following security requirements have been identified by SA3:

- REQ 1: An IMS service provider shall ensure that a third party authentication service for WebRTC IMS Clients (WIC) authenticating and authorizing an IMS subscriber attempting to register with IMS network using one of the IMS subscriber’s own IMS identities are granted the right to do so by the IMS subscriber. In case of a potential security breach affecting that third party, IMS subscribers that did not grant any right to that third party shall not be affected.

- REQ 2: An IMS service provider should be able to identify and mitigate security anomalies or security breaches at one entity providing a third party authentication service selectively, without affecting clients associated with other entities providing a third party authentication service.

Editor’s Note: This clause will define additional potential security requirements.

*** END CHANGES ***