3GPP TSG SA WG3 (Security) Meeting #79	S3-151349

20-24 April 2015, Nanjing (China)		revision of none

Source:	Nokia Networks

Title:	pCR to TR 33.806 on GTP-C filtering requirements

Document for:	Discussion and Decision

Agenda Item:	7.11

Work Item / Release:	Rel-13

Abstract of the contribution:

The present contribution proposes text on GTP-C filtering to be included in clause B.3.7.2.

## 1 Introduction

The present contribution does not intend to cover the entire clause B.3.7.2 “Protecting availability and integrity”, but only the aspect of GTP-C filtering. It is part of a larger discussion conducted over the SA3 list and at SCAS conf calls.

The main idea behind the present contribution is that filtering functions may be performed on the MME itself or on a firewall in front of it. It does not matter so much where the filtering functions are performed, but that it is guaranteed that they are performed at all. In order to capture this situation the notion of ‘conditional requirement’ is introduced.

# 2 New text for Annex B.3.7.2 “Protecting availability and integrity”

## +++START OF CHANGES +++

#### B.3.7.2 	Protecting availability and integrity

Editor’s note: relates to Req 3.42-15 in DT’s catalogue, no equivalent in main body of TR.

##### B.3.7.2.x	GTP-C Filtering

Requirement Name: GTP-C Filtering

Requirement Reference: to be done later

Requirement Description:

The following capability is conditionally required:

- For each message of a GTP-C-based protocol, it shall be possible to check whether the sender of this message is authorised to send a message pertaining to this protocol.

This requirement is conditional in the following sense: It is required that at least one of the following two statements holds:

- The MME supports the capability described above.

- A separate entity provides the capability described above, and the MME product documentation states that the MME needs to be deployed together with such a separate entity.

NOTE 1: Such a separate entity could e.g. be a GTP Firewall.

NOTE 2: Test cases for this separate entity are not provided in the present document, but are believed to be similar to them.

NOTE 3: The check could be performed e.g. against a whitelist or blacklist of permitted message type / sender identity combinations.

Threat References: tba

Security Objective references: tba.

Test case: TBD

## +++END OF CHANGES to Annex B +++