3GPP TSG SA WG3 (Security) Meeting #79bis	S3-151670

Sophia Antipolis, France, 29 – 30 June 2015	revision of S3-151620

Source:	Nokia Networks

Title:	Mapping TR 33.806, B.4.5, to TS 33.116 and TS 33.117

Document for:	Discussion/approval

Agenda Item:	4

Work Item / Release:	SCAS / Rel-13

Abstract of the contribution:

It was agreed at SA3#79 to include MME-specific requirements into TS 33.116 and generic requirements into TS 33.117. It was further agreed to take clauses from TR 33.806 and map them to the corresponding clauses of the two TSs. The present contribution maps TR 33.806, B.4.5, to TS 33.116 and TS 33.117.

The first section of the contribution copies the text from TR 33.806, B.4.5 (there were no changes during the conf calls), and adds Word comments explaining which part of the text is believed to be MME-specific and which is generic. It turns out that, for this clause, there are no MME-specific parts. The existing Word comments from TR 33.806 were removed for better readability of the new comments.

The second section of the contribution provides a pCR to TS 33.117, v0.0.3.

The third section of the contribution provides a pCR to TS 33.116, v0.0.3.

In the present contribution, only the requirements themselves are mapped, not the test cases.

The technical content in S3-151670 does not differ from that in S3-151620.

# Annotated text from TR 33.806

### B.4.5	Web Servers

#### B.4.5.1	No system privileges for web server

Requirement Name: All web server processes shall not run with system privileges.

Requirement Reference: TBA

Requirement Description:

All web server processes shall not run with system privileges. If a process is started by a user with system privileges, execution shall be transferred to a different user without system privileges after the start.

Threat References: TBA

Security Objective References: TBA

Test Case: TBA

#### B.4.5.2	No unused HTTP methods

Requirement Name: HTTP methods that are not required shall be deactivated.

Requirement Reference: TBA

Requirement Description:

HTTP methods that are not required shall be deactivated. Standard requests to web servers only use GET and POST. If other methods are required, they shall not introduce security leaks such as TRACK or TRACE).

Threat References: TBA

Security Objective References: TBA

Test Case: TBA

#### B.4.5.3	No unused add-ons

Requirement Name: Any add-ons and components that are not required shall be deactivated.

Requirement Reference: TBA

Requirement Description: All optional add-ons and components of the web server shall be deactivated if they are not required. In particular, CGI, Server Side Includes (SSI) and WebDAV shall be deactivated if they are not required.

Threat References: TBA

Security Objective References: TBA

Test Case: TBA

#### B.4.5.4	No compiler, interpreter, or shell via CGI

Requirement Name: If CGI (Common Gateway Interface) is used, the CGI directory shall not include compilers or interpreters (e.g., PERL interpreter, PHP interpreter/compiler, Tcl interpreter/compiler or operating system shells).

Requirement Reference: TBA

Requirement Description: : If CGI (Common Gateway Interface) is used, the CGI directory shall not include compilers or interpreters (e.g., PERL interpreter, PHP interpreter/compiler, Tcl interpreter/compiler or operating system shells).

Threat References: TBA

Security Objective References: TBA

Test Case: TBA

#### B.4.5.5	No CGI uploads

Requirement Name: If CGI is used, the CGI directory shall not be used for uploads.

Requirement Reference: TBA

Requirement Description: If CGI is used, the CGI directory shall not be used for uploads.

Threat References: TBA

Security Objective References: TBA

Test Case: TBA

#### B.4.5.6	No execution of system commands

Requirement Name: If Server Side Includes (SSI) are active, the execution of system commands shall be deactivated.

Requirement Reference: TBA

Requirement Description: If Server Side Includes (SSI) are active, the execution of system commands shall be deactivated.

Threat References: TBA

Security Objective References: TBA

Test Case: TBA

#### B.4.5.7	Access rights for web server configuration

Requirement Name: Access rights for web server configuration files shall only be granted to the owner of the web server process or a user with system privileges.

Requirement Reference: TBA

Requirement Description: Access rights for web server configuration files shall only be granted to the owner of the web server process or a user with system privileges. Implementation example: Delete “read” and “write” access rights for “others.” Only grant “write” access to the user who configures the web server..

Threat References: TBA

Security Objective References: TBA

Test Case: TBA

#### B.4.5.8	No default content

Requirement Name: Default content shall be removed.

Requirement Reference: TBA

Requirement Description: Default content (examples, help files, documentation, aliases) that is provided with the standard installation of the web server shall be removed.

Threat References: TBA

Security Objective References: TBA

Test Case: TBA

#### B.4.5.9	No directory listings

Requirement Name: Directory listings (indexing) shall be deactivated.

Requirement Reference: TBA

Requirement Description: Directory listings (indexing) shall be deactivated.

Threat References: TBA

Security Objective References: TBA

Test Case: TBA

#### B.4.5.10	Web server information in HTTP headers

Requirement Name: Information about the web server in HTTP headers shall be minimized.

Requirement Reference: TBA

Requirement Description: The HTTP header shall not include information on the version of the web server and the modules/add-ons used.

Threat References: TBA

Security Objective References: TBA

Test Case: TBA

#### B.4.5.11	Web server information in error pages

Requirement Name: Web server information in error pages shall be deleted. Default error pages of the web server shall be replaced by error pages defined by the TOE vendor.

Requirement Reference: TBA

Requirement Description: User-defined error pages shall not include version information about the web server and the modules/addons used. Error messages shall not include internal information such as internal server names, error codes, etc.

Threat References: TBA

Security Objective References: TBA

Test Case: TBA

#### B.4.5.12	Minimised file type mappings

Requirement Name: File type or script mappings that are not required shall be deleted

Requirement Reference: TBA

Requirement Description: File type or script mappings that are not required shall be deleted, e.g., php, phtml, js, sh, csh, bin, exe, pl, vbe, vbs.

Threat References: TBA

Security Objective References: TBA

Test Case: TBA

#### B.4.5.13	Restricted file access

Requirement Name: The web server shall only deliver files which are meant to be delivered.

Requirement Reference: TBA

Requirement Description: Restrictive access rights shall be assigned to all files which are directly or indirectly (e.g., via links or in virtual directories) in the web server’s document directory. In particular, the web server shall not be able to access files which are not meant to be delivered.

Threat References: TBA

Security Objective References: TBA

Test Case: TBA

# pCR to TS 33.117 (generic requirements)

***	START OF CHANGE	***

### 5.3.4	Web Servers

#### 5.3.4.1	No system privileges for web server

Requirement Name: All web server processes shall not run with system privileges.

Requirement Reference: TBA

Requirement Description:

All web server processes shall not run with system privileges. If a process is started by a user with system privileges, execution shall be transferred to a different user without system privileges after the start.

Threat References: TBA

Security Objective References: TBA

#### Test Case: TBA

#### 5.3.4.2	No unused HTTP methods

Requirement Name: HTTP methods that are not required shall be deactivated.

Requirement Reference: TBA

Requirement Description:

HTTP methods that are not required shall be deactivated. Standard requests to web servers only use GET and POST. If other methods are required, they shall not introduce security leaks such as TRACK or TRACE).

Threat References: TBA

#### Security Objective

#### 5.3.4.3	No unused add-ons

Requirement Name: Any add-ons and components that are not required shall be deactivated.

Requirement Reference: TBA

Requirement Description: All optional add-ons and components of the web server shall be deactivated if they are not required. In particular, CGI, Server Side Includes (SSI) and WebDAV shall be deactivated if they are not required.

Threat References: TBA

Security Objective References: TBA

#### Test Case: TBA

#### 5.3.4.4	No compiler, interpreter, or shell via CGI

Requirement Name: If CGI (Common Gateway Interface) is used, the CGI directory shall not include compilers or interpreters (e.g., PERL interpreter, PHP interpreter/compiler, Tcl interpreter/compiler or operating system shells).

Requirement Reference: TBA

Requirement Description: : If CGI (Common Gateway Interface) is used, the CGI directory shall not include compilers or interpreters (e.g., PERL interpreter, PHP interpreter/compiler, Tcl interpreter/compiler or operating system shells).

Threat References: TBA

Security Objective References: TBA

#### Test Case: TBA

#### 5.3.4.5	No CGI uploads

Requirement Name: If CGI is used, the CGI directory shall not be used for uploads.

Requirement Reference: TBA

Requirement Description: If CGI is used, the CGI directory shall not be used for uploads.

Threat References: TBA

Security Objective References: TBA

#### Test Case: TBA

#### 5.3.4.6	No execution of system commands

Requirement Name: If Server Side Includes (SSI) are active, the execution of system commands shall be deactivated.

Requirement Reference: TBA

Requirement Description: If Server Side Includes (SSI) are active, the execution of system commands shall be deactivated.

Threat References: TBA

#### Security Objective References: TBA

#### Test Case: TBA

#### 5.3.4.7	Access rights for web server configuration

Requirement Name: Access rights for web server configuration files shall only be granted to the owner of the web server process or a user with system privileges.

Requirement Reference: TBA

Requirement Description: Access rights for web server configuration files shall only be granted to the owner of the web server process or a user with system privileges. Implementation example: Delete “read” and “write” access rights for “others.” Only grant “write” access to the user who configures the web server..

Threat References: TBA

Security Objective References: TBA

#### Test Case: TBA

#### 5.3.4.8	No default content

Requirement Name: Default content shall be removed.

Requirement Reference: TBA

Requirement Description: Default content (examples, help files, documentation, aliases) that is provided with the standard installation of the web server shall be removed.

Threat References: TBA

Security Objective References: TBA

#### Test Case: TBA

#### 5.3.4.9	No directory listings

Requirement Name: Directory listings (indexing) shall be deactivated.

Requirement Reference: TBA

Requirement Description: Directory listings (indexing) shall be deactivated.

Threat References: TBA

Security Objective References: TBA

#### Test Case: TBA

#### 5.3.4.10	Web server information in HTTP headers

Requirement Name: Information about the web server in HTTP headers shall be minimized.

Requirement Reference: TBA

Requirement Description: The HTTP header shall not include information on the version of the web server and the modules/add-ons used.

Threat References: TBA

Security Objective References: TBA

#### Test Case: TBA

#### 5.3.4.11	Web server information in error pages

Requirement Name: Web server information in error pages shall be deleted. Default error pages of the web server shall be replaced by error pages defined by the TOE vendor.

Requirement Reference: TBA

Requirement Description: User-defined error pages shall not include version information about the web server and the modules/addons used. Error messages shall not include internal information such as internal server names, error codes, etc.

Threat References: TBA

#### Security Objective References: TBA

#### Test Case: TBA

#### 5.3.4.12	Minimised file type mappings

Requirement Name: File type or script mappings that are not required shall be deleted

Requirement Reference: TBA

Requirement Description: File type or script mappings that are not required shall be deleted, e.g., php, phtml, js, sh, csh, bin, exe, pl, vbe, vbs.

Threat References: TBA

Security Objective References: TBA

#### Test Case: TBA

#### 5.3.4.13	Restricted file access

Requirement Name: The web server shall only deliver files which are meant to be delivered.

Requirement Reference: TBA

Requirement Description: Restrictive access rights shall be assigned to all files which are directly or indirectly (e.g., via links or in virtual directories) in the web server’s document directory. In particular, the web server shall not be able to access files which are not meant to be delivered.

Threat References: TBA

Security Objective References: TBA

Test Case: TBA

***	END OF CHANGE	***

# pCR to TS 33.116 (MME-specific adaptations of generic requirements from TS 33.117)

***	START OF CHANGE	***

### 5.3.4	Web Servers

#### There are no MME-specific additions to clause 5.3.4 of TS 33.117.

***	END OF CHANGE	***